# Module 1

Q1. Explain the OSI Reference Model?

The OSI (Open Systems Interconnection) reference model is a conceptual framework developed by ISO that standardizes how different network systems communicate. It divides network communication into 7 distinct layers, each with specific responsibilities.

| Layer | Name | Function |
| --- | --- | --- |
| 7 | Application | User interface; HTTP, FTP, SMTP, DNS |
| 6 | Presentation | Data formatting, encryption, compression |
| 5 | Session | Manages sessions/connections between applications |
| 4 | Transport | End-to-end delivery; TCP, UDP; segmentation |
| 3 | Network | Routing, IP addressing; logical addressing |
| 2 | Data Link | MAC addressing, framing, error detection (Ethernet) |
| 1 | Physical | Transmission of raw bits over physical medium |

Each layer communicates with the layer directly above and below it, adding its own header/trailer (encapsulation) when sending and stripping them (de-encapsulation) when receiving.

Q2. What is a Network?

A network is a collection of two or more interconnected computing devices (computers, servers, printers, routers, etc.) that can communicate and share resources with each other.

Key characteristics of a network:

- Resource Sharing: Files, printers, internet connections can be shared

- Communication: Devices can exchange data using protocols

- Scalability: Networks can grow from 2 devices to millions

- Types: LAN (Local Area Network), WAN (Wide Area Network), MAN (Metropolitan Area Network)

Q3. What are Routers?

A router is a network device that operates at Layer 3 (Network layer) of the OSI model. It connects multiple networks and directs data packets between them based on IP addresses.

Key functions of a router:

- Path determination: Finds the best path to a destination network

- Packet forwarding: Forwards packets based on routing table entries

- Network segmentation: Separates broadcast domains

- Protocol translation: Can connect networks using different protocols

- NAT: Translates private IPs to public IPs for internet access

Routers use routing tables and routing protocols (RIP, OSPF, EIGRP) to make forwarding decisions.

Q4. Explain Encapsulation.

Encapsulation is the process of adding protocol-specific headers (and sometimes trailers) to data as it travels down the OSI layers from the Application layer to the Physical layer.

How encapsulation works at each layer:

- Layer 7-5 (Application/Presentation/Session): Data is created by the application

- Layer 4 (Transport): Data is segmented; TCP/UDP header is added -> called Segment

- Layer 3 (Network): IP header (source/destination IP) is added -> called Packet

- Layer 2 (Data Link): MAC header + trailer is added -> called Frame

- Layer 1 (Physical): Frame is converted to bits for transmission

De-encapsulation is the reverse process at the receiving end, where each layer strips its respective header.

Q5. Peer-to-Peer Communication.

Peer-to-peer (P2P) communication is a network model where each device (peer) can act as both a client and a server. In P2P, devices communicate directly with each other without a central server.

Characteristics:

- Each peer has equal status and responsibilities

- Resources (files, bandwidth) are shared directly between peers

- No central point of control or failure

- Cost-effective for small networks (no dedicated server needed)

Comparison with Client-Server model:

| Aspect | Peer-to-Peer | Client-Server |
| --- | --- | --- |
| Control | Decentralized | Centralized server |
| Scalability | Limited | Highly scalable |
| Security | Harder to manage | Easier to manage |
| Cost | Low | Higher (server needed) |
| Example | BitTorrent, Skype | Web servers, Email servers |

Q6. What is TCP and UDP?

TCP (Transmission Control Protocol) and UDP (User Datagram Protocol) are both Layer 4 (Transport) protocols used to send data over networks.

TCP (Transmission Control Protocol):

- Connection-oriented: establishes a 3-way handshake before data transfer (SYN, SYN-ACK, ACK)

- Reliable: guarantees delivery, ordering, and error-checking

- Slower due to overhead of acknowledgments

- Used for: HTTP/HTTPS, FTP, SMTP, SSH

UDP (User Datagram Protocol):

- Connectionless: no handshake, just sends data

- Unreliable: no guarantee of delivery or ordering

- Faster due to minimal overhead

- Used for: DNS, DHCP, video streaming, VoIP, gaming

| Feature | TCP | UDP |
| --- | --- | --- |
| Connection | Connection-oriented | Connectionless |
| Reliability | Reliable (ACK) | Unreliable |
| Speed | Slower | Faster |
| Error checking | Yes | Optional |
| Use case | File transfer, web | Streaming, DNS |

Q7. What is Internetwork Operating System (IOS) Software?

Cisco IOS (Internetwork Operating System) is the proprietary operating system used on most Cisco routers and switches. It provides a CLI (Command Line Interface) for configuring, managing, and monitoring network devices.

Key features of Cisco IOS:

- CLI-based configuration using exec and configuration modes

- Supports routing protocols: RIP, EIGRP, OSPF, BGP

- Supports switching features: VLANs, STP, VTP

- Security features: ACLs, AAA, password encryption

- Monitoring: show commands, debug commands, logging

IOS Modes:

- User EXEC Mode (>): Limited view-only commands

- Privileged EXEC Mode (#): Full monitoring and management

- Global Configuration Mode (config)#: System-wide configuration

- Interface Configuration Mode (config-if)#: Interface-specific settings

Q8. Explain LAN and Draw any Example.

A LAN (Local Area Network) is a network that connects devices within a limited geographic area such as a home, school, office building, or campus.

Characteristics of LAN:

- Covers a small geographic area (up to a few km)

- High data transfer speeds (100 Mbps to 10 Gbps)

- Privately owned and managed

- Uses Ethernet (IEEE 802.3) or Wi-Fi (IEEE 802.11)

- Low error rates due to short distances

Example LAN Topology:

[PC1] ---+

|

[PC2] ---+--- [Switch] --- [Router] --- [Internet]

|

[PC3] ---+

|

[Printer]+

In this example, all PCs and the printer are connected to a central switch, which connects to a router for internet access.

Q9. Explain Network Devices – Router, Switch, and Hub.

| Device | OSI Layer | Function | Key Feature |
| --- | --- | --- | --- |
| Hub | Layer 1 (Physical) | Broadcasts data to all ports | Dumb device; no intelligence; creates collisions |
| Switch | Layer 2 (Data Link) | Forwards frames using MAC address table | Intelligent; reduces collisions; creates separate collision domains |
| Router | Layer 3 (Network) | Routes packets between networks using IP addresses | Connects different networks; uses routing table; creates broadcast domains |

- Hub: All devices receive data even if not the intended recipient. Obsolete today.

- Switch: Learns MAC addresses and forwards data only to the correct port. Efficient.

- Router: Connects LAN to WAN (Internet). Makes routing decisions using IP.

Q10. Describe Router and Switch Connection in LAN.

In a LAN, switches and routers work together:

Switch role in LAN:

- Connects all end devices (PCs, printers, servers) within the LAN

- Operates at Layer 2 using MAC addresses

- Provides high-speed switching between devices

Router role in LAN:

- Acts as the default gateway for the LAN

- Connects the LAN to external networks (WAN/Internet)

- Performs NAT, DHCP relay, and inter-VLAN routing

Typical Connection:

PC1, PC2, PC3 -> [Switch Port Fa0/1, Fa0/2, Fa0/3]

Switch Fa0/24 (Uplink) -> Router Fa0/0 (LAN interface)

Router Fa0/1 (WAN interface) -> ISP/Internet

Q11. Types of Cable – Explain Types of Ethernets and Speed.

Ethernet cables are categorized by their category (Cat) ratings and support different speeds:

| Cable Type | Standard | Max Speed | Max Distance |
| --- | --- | --- | --- |
| Cat5 | 100BASE-TX | 100 Mbps | 100 meters |
| Cat5e | 1000BASE-T | 1 Gbps | 100 meters |
| Cat6 | 1000BASE-T / 10GBASE-T | 1–10 Gbps | 55–100 meters |
| Cat6a | 10GBASE-T | 10 Gbps | 100 meters |
| Cat7 | 10GBASE-T | 10 Gbps | 100 meters |
| Fiber (Single-mode) | 1G/10G Ethernet | 1–100 Gbps | Up to 100 km |
| Fiber (Multi-mode) | 1G/10G Ethernet | 1–10 Gbps | Up to 2 km |

Cable Types:

- Straight-through cable: Connects different devices (PC to Switch, Switch to Router)

- Crossover cable: Connects similar devices (PC to PC, Switch to Switch)

- Rollover/Console cable: Connects a PC to router/switch console port for management

Q12. Explain TCP/IP – List of Protocols and Port Numbers.

TCP/IP is the foundational protocol suite of the Internet. It maps to a 4-layer model.

| Protocol | Port | Transport | Description |
| --- | --- | --- | --- |
| HTTP | 80 | TCP | Web browsing |
| HTTPS | 443 | TCP | Secure web browsing |
| FTP | 20/21 | TCP | File transfer |
| SSH | 22 | TCP | Secure remote access |
| Telnet | 23 | TCP | Remote access (unsecure) |
| SMTP | 25 | TCP | Sending email |
| DNS | 53 | UDP/TCP | Domain name resolution |
| DHCP | 67/68 | UDP | IP address assignment |
| TFTP | 69 | UDP | Trivial file transfer |
| POP3 | 110 | TCP | Receiving email |
| IMAP | 143 | TCP | Email access |
| SNMP | 161 | UDP | Network management |
| HTTPS | 443 | TCP | Secure HTTP |
| RIP | 520 | UDP | Routing protocol |

Q13. Explain Node (Backbone) and Physical Layer.

Backbone Node:

A backbone (or core) node is a high-speed, high-capacity network device or link that carries aggregated traffic from multiple edge networks. It forms the core of a network infrastructure.

- Backbone switches/routers handle large volumes of data

- They interconnect distribution-layer devices

- Examples: Core routers in ISPs, campus core switches

Physical Layer (OSI Layer 1):

The Physical layer is the lowest layer of the OSI model. It is responsible for the transmission and reception of raw bit streams over a physical medium.

- Defines electrical, optical, and mechanical specifications

- Deals with cables, connectors, NICs, hubs, repeaters

- Specifies voltage levels, data rates, signal timing

- Physical media: copper wire, fiber optic, radio waves

# Module 2

Q1. Describe IPv4 Address Range and Explain Example of Subnetting.

IPv4 uses 32-bit addresses written in dotted-decimal notation (e.g., 192.168.1.1). Total possible addresses: 2^32 = ~4.3 billion.

IPv4 Address Classes:

| Class | Range | Default Subnet Mask | Usage |
| --- | --- | --- | --- |
| A | 1.0.0.0 – 126.255.255.255 | 255.0.0.0 (/8) | Large organizations |
| B | 128.0.0.0 – 191.255.255.255 | 255.255.0.0 (/16) | Medium organizations |
| C | 192.0.0.0 – 223.255.255.255 | 255.255.255.0 (/24) | Small organizations |
| D | 224.0.0.0 – 239.255.255.255 | N/A | Multicast |
| E | 240.0.0.0 – 255.255.255.255 | N/A | Reserved/Research |

Subnetting Example:

Given network: 192.168.1.0/24 – Divide into 4 subnets.

We need 2 extra bits from the host portion: /24 + 2 = /26

- Subnet 1: 192.168.1.0/26   | Hosts: .1 – .62   | Broadcast: .63

- Subnet 2: 192.168.1.64/26  | Hosts: .65 – .126 | Broadcast: .127

- Subnet 3: 192.168.1.128/26 | Hosts: .129 – .190 | Broadcast: .191

- Subnet 4: 192.168.1.192/26 | Hosts: .193 – .254 | Broadcast: .255

Each subnet supports 62 usable hosts (2^6 - 2 = 62).

Q2. List of Private Addresses.

Private IP addresses are reserved for internal use and are not routable on the public internet.

| Class | Private Range | Subnet Mask | No. of Addresses |
| --- | --- | --- | --- |
| A | 10.0.0.0 – 10.255.255.255 | 255.0.0.0 (/8) | 16,777,216 |
| B | 172.16.0.0 – 172.31.255.255 | 255.240.0.0 (/12) | 1,048,576 |
| C | 192.168.0.0 – 192.168.255.255 | 255.255.0.0 (/16) | 65,536 |

Other special addresses: 127.0.0.0/8 (loopback), 169.254.0.0/16 (APIPA), 0.0.0.0 (default route).

Q3. What is Routing? Explain Work of Router and Protocol.

Routing is the process of selecting the best path for data packets to travel from a source to a destination across one or more networks.

How a Router works:

- Receives a packet on an interface

- Examines the destination IP address

- Consults its routing table to find the best match

- Forwards the packet out the appropriate interface

- If no match found, uses default route or drops the packet

Routing Protocols help routers share network information:

- Distance Vector: RIP – shares routing table with neighbors periodically

- Link State: OSPF – shares link state information with all routers

- Hybrid: EIGRP – combines distance vector and link state features

Q4. Which Software We Use for Routing and Switching?

The most commonly used simulation software for learning Cisco routing and switching:

- Cisco Packet Tracer: Free simulator by Cisco; ideal for CCNA practice; supports routers, switches, wireless devices

- GNS3 (Graphical Network Simulator): Uses actual Cisco IOS images; more realistic; used by professionals

- EVE-NG: Advanced emulation platform; runs real network OS images; used in enterprise labs

- Cisco VIRL/CML: Cisco's official modeling lab; subscription-based

For this course, Cisco Packet Tracer is the primary tool used.

Q5. Explain Basic Commands.

Basic Cisco IOS commands used on routers and switches:

Router> enable                          ! Enter privileged exec mode

Router# configure terminal              ! Enter global config mode

Router(config)# hostname R1             ! Set device hostname

Router(config)# enable secret cisco123  ! Set encrypted enable password

Router(config)# interface fa0/0          ! Enter interface config

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# no shutdown           ! Enable the interface

Router# show ip interface brief          ! View interface summary

Router# show running-config              ! View current config

Router# copy running-config startup-config  ! Save config

Q6. Types of Routing – Example of Static Routing.

Types of Routing:

- Static Routing: Manually configured routes; no protocol overhead; best for small/stable networks

- Dynamic Routing: Routes learned automatically via protocols (RIP, OSPF, EIGRP)

- Default Routing: A catch-all route (0.0.0.0/0) used when no specific match is found

Static Routing Example:

Topology: R1 (192.168.1.0/24) <-> R2 <-> R3 (192.168.3.0/24)

! On R1: To reach 192.168.3.0 network via R2 (10.0.0.2)

R1(config)# ip route 192.168.3.0 255.255.255.0 10.0.0.2

! On R3: To reach 192.168.1.0 network via R2 (10.0.0.1)

R3(config)# ip route 192.168.1.0 255.255.255.0 10.0.0.1

Q7. Explain Dynamic Routing.

Dynamic routing allows routers to automatically discover and maintain routing information by exchanging routing updates with neighboring routers.

Advantages:

- Automatically adapts to network topology changes

- Reduces manual configuration in large networks

- Provides redundancy and load balancing

Types of Dynamic Routing Protocols:

- Distance Vector (RIP): Uses hop count as metric; shares full routing table

- Link State (OSPF): Builds complete topology map; uses Dijkstra's SPF algorithm

- Hybrid (EIGRP): Uses DUAL algorithm; combines best of both

Q8. Difference Between RIP, EIGRP, and OSPF.

| Feature | RIP | EIGRP | OSPF |
| --- | --- | --- | --- |
| Type | Distance Vector | Hybrid (Advanced DV) | Link State |
| Metric | Hop count (max 15) | Bandwidth, Delay, Load, Reliability | Cost (based on bandwidth) |
| Algorithm | Bellman-Ford | DUAL | Dijkstra (SPF) |
| Admin Distance | 120 | 90 (internal) | 110 |
| Convergence | Slow | Fast | Fast |
| Scalability | Small networks | Medium to large | Large networks |
| Protocol | UDP port 520 | IP protocol 88 | IP protocol 89 |
| Vendor | Open standard | Cisco proprietary | Open standard |

Q9. Perform Example of RIP, EIGRP, and OSPF with Different Area Concept.

RIP Configuration Example:

Router(config)# router rip

Router(config-router)# version 2

Router(config-router)# network 192.168.1.0

Router(config-router)# network 10.0.0.0

Router(config-router)# no auto-summary

EIGRP Configuration Example:

Router(config)# router eigrp 100          ! AS number 100

Router(config-router)# network 192.168.1.0 0.0.0.255

Router(config-router)# network 10.0.0.0 0.0.0.255

Router(config-router)# no auto-summary

OSPF Configuration Example (with areas):

Router(config)# router ospf 1             ! Process ID 1

Router(config-router)# network 192.168.1.0 0.0.0.255 area 0  ! Backbone area

Router(config-router)# network 10.0.0.0 0.0.0.255 area 1    ! Area 1

OSPF Area Concept: Area 0 is the backbone. All other areas must connect to Area 0. ABRs (Area Border Routers) connect multiple areas.

Q10. Example of Default Routing.

Default routing is used when no specific route matches the destination. It is the route of last resort, typically used on edge routers connecting to the internet.

! Default route pointing to ISP (gateway 203.0.113.1)

Router(config)# ip route 0.0.0.0 0.0.0.0 203.0.113.1

! For OSPF, redistribute default route:

Router(config-router)# default-information originate

! Verify:

Router# show ip route

! Look for: S* 0.0.0.0/0 [1/0] via 203.0.113.1

Q11. Explain Autonomous System Number.

An Autonomous System (AS) is a collection of IP networks and routers under the control of a single organization that presents a common routing policy to the internet.

Key points:

- Each AS is identified by a unique Autonomous System Number (ASN)

- ASNs range from 1 to 65535 (16-bit) or 1 to 4294967295 (32-bit)

- Public ASNs (1-64511): Used on the internet, assigned by IANA/RIRs

- Private ASNs (64512-65535): Used internally, not advertised on internet

- BGP (Border Gateway Protocol) uses ASNs to route between autonomous systems

- EIGRP also uses AS numbers, but for internal identification only

! EIGRP with AS 100:

Router(config)# router eigrp 100

! BGP with AS 65001:

Router(config)# router bgp 65001

Q12. What is Switching? Explain VLAN.

Switching is the process of forwarding frames within a network based on MAC addresses. A switch builds a MAC address table (CAM table) by learning the source MAC of incoming frames and uses it to forward frames only to the correct destination port.

VLAN (Virtual Local Area Network):

A VLAN is a logical grouping of devices on a switch regardless of their physical location. VLANs segment a single physical switch into multiple virtual switches.

Benefits of VLANs:

- Security: Isolates traffic between departments

- Performance: Reduces broadcast domain size

- Flexibility: Devices can be grouped logically, not physically

- Management: Easier network management

Switch(config)# vlan 10

Switch(config-vlan)# name SALES

Switch(config)# vlan 20

Switch(config-vlan)# name HR

Q13. What is Access Port and Trunk Port?

| Feature | Access Port | Trunk Port |
| --- | --- | --- |
| Belongs to | Single VLAN | Multiple VLANs |
| Frame type | Untagged Ethernet frames | 802.1Q tagged frames |
| Connected to | End devices (PCs, printers) | Other switches, routers |
| VLAN tagging | No tagging | Tagged with VLAN ID |
| Use case | User devices | Inter-switch / router links |

! Access port configuration:

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 10

! Trunk port configuration:

Switch(config-if)# switchport mode trunk

Switch(config-if)# switchport trunk encapsulation dot1q

Q14. List of Basic SHOW Commands.

| Command | Description |
| --- | --- |
| show ip interface brief | Displays a summary of all interfaces with IP and status |
| show running-config | Displays the current active configuration |
| show startup-config | Displays the saved configuration in NVRAM |
| show ip route | Displays the routing table |
| show version | Displays IOS version, hardware info, uptime |
| show interfaces | Detailed info on all interfaces |
| show vlan brief | Displays VLAN summary on a switch |
| show mac address-table | Shows learned MAC addresses on switch |
| show ip protocols | Shows active routing protocols |
| show cdp neighbors | Shows directly connected Cisco devices |
| show spanning-tree | Displays STP information |
| show ip ospf neighbor | Shows OSPF neighbor relationships |
| show ip eigrp neighbors | Shows EIGRP neighbor table |

Q15. Explain Layer 2 and Layer 3 Switch.

| Feature | Layer 2 Switch | Layer 3 Switch |
| --- | --- | --- |
| OSI Layer | Layer 2 (Data Link) | Layer 2 + Layer 3 (Network) |
| Forwarding basis | MAC addresses | MAC + IP addresses |
| Routing | Cannot route | Can perform inter-VLAN routing |
| Routing table | No routing table | Has routing table (like router) |
| Cost | Lower | Higher |
| Use case | Within a VLAN | Between VLANs, distribution layer |

! Enable routing on Layer 3 switch:

Switch(config)# ip routing

Switch(config)# interface vlan 10

Switch(config-if)# ip address 192.168.10.1 255.255.255.0

Q16. Example – VLAN Access Port and Trunk Port.

Scenario: SW1 has VLAN 10 (Sales) and VLAN 20 (HR). PC1 is in VLAN 10, PC2 is in VLAN 20. SW1 connects to SW2 via trunk.

! On SW1:

SW1(config)# vlan 10

SW1(config-vlan)# name SALES

SW1(config)# vlan 20

SW1(config-vlan)# name HR

! PC1 connected to Fa0/1 -> VLAN 10 access port

SW1(config)# interface fa0/1

SW1(config-if)# switchport mode access

SW1(config-if)# switchport access vlan 10

! PC2 connected to Fa0/2 -> VLAN 20 access port

SW1(config)# interface fa0/2

SW1(config-if)# switchport mode access

SW1(config-if)# switchport access vlan 20

! Trunk between SW1 and SW2 on Gi0/1

SW1(config)# interface gi0/1

SW1(config-if)# switchport mode trunk

Q17. Example of Inter-VLAN Routing.

Inter-VLAN routing allows communication between different VLANs. It can be done using a router-on-a-stick or a Layer 3 switch.

Router-on-a-Stick Method:

! Router sub-interface configuration:

Router(config)# interface fa0/0.10

Router(config-subif)# encapsulation dot1Q 10

Router(config-subif)# ip address 192.168.10.1 255.255.255.0

Router(config)# interface fa0/0.20

Router(config-subif)# encapsulation dot1Q 20

Router(config-subif)# ip address 192.168.20.1 255.255.255.0

Router(config)# interface fa0/0

Router(config-if)# no shutdown

Q18. Explain Switching Method and VTP.

Switching Methods:

- Store-and-Forward: Receives entire frame, checks for errors (CRC), then forwards. Most reliable.

- Cut-Through: Forwards frame after reading only the destination MAC. Fastest, no error checking.

- Fragment-Free: Reads first 64 bytes (avoids collision fragments), then forwards. Compromise.

VTP (VLAN Trunking Protocol):

VTP is a Cisco proprietary protocol that allows automatic synchronization of VLAN information across all switches in a VTP domain.

VTP Modes:

- Server: Can create, modify, delete VLANs; advertises VTP info

- Client: Receives VTP updates; cannot create VLANs locally

- Transparent: Does not participate in VTP; forwards VTP messages but uses local VLAN database

Switch(config)# vtp domain CCNA

Switch(config)# vtp mode server

Switch(config)# vtp password cisco

Q19. What is Spanning Tree – Mention Spanning Tree Protocol and Algorithm?

Spanning Tree Protocol (STP) is a Layer 2 protocol (IEEE 802.1D) that prevents switching loops in networks with redundant links.

STP Algorithm (Steps):

- Step 1: Elect a Root Bridge – the switch with the lowest Bridge ID (Priority + MAC)

- Step 2: Each non-root switch selects a Root Port (closest port to root bridge)

- Step 3: Each network segment selects a Designated Port (best port on that segment)

- Step 4: All other ports are placed in Blocking state

STP Port States:

- Blocking -> Listening -> Learning -> Forwarding (or Disabled)

STP Variants:

- 802.1D (Classic STP): Original, slow convergence (~50 seconds)

- RSTP (802.1w): Rapid STP; faster convergence

- PVST+: Cisco Per-VLAN Spanning Tree

- MSTP (802.1s): Multiple Spanning Tree

Q20. Example of Per VLAN Spanning Tree.

Per VLAN Spanning Tree Plus (PVST+) runs a separate STP instance for each VLAN, allowing different root bridges per VLAN for load balancing.

! Set SW1 as root for VLAN 10, SW2 as root for VLAN 20:

SW1(config)# spanning-tree vlan 10 priority 4096   ! Lower = preferred root

SW1(config)# spanning-tree vlan 20 priority 8192

SW2(config)# spanning-tree vlan 10 priority 8192

SW2(config)# spanning-tree vlan 20 priority 4096

! Verify:

SW1# show spanning-tree vlan 10

Q21. What is IPv6? Explain Types and IP Address Range.

IPv6 is the successor to IPv4, using 128-bit addresses to solve IPv4 address exhaustion. Written in 8 groups of 4 hex digits separated by colons: e.g., 2001:0DB8:85A3:0000:0000:8A2E:0370:7334

IPv6 Address Types:

| Type | Prefix | Scope | Description |
| --- | --- | --- | --- |
| Unicast - Global | 2000::/3 | Internet | Routable on public internet (like public IPv4) |
| Unicast - Link-Local | FE80::/10 | Single link | Auto-configured; used for local communication |
| Unicast - Loopback | ::1/128 | Local host | Equivalent to 127.0.0.1 in IPv4 |
| Multicast | FF00::/8 | Various | One-to-many; replaces IPv4 broadcast |
| Anycast | From unicast space | Network | One-to-nearest; same address on multiple interfaces |
| Unspecified | ::/128 | N/A | Used when no address is assigned |

Q22. Example of IPv6 – RIP (RIPng).

RIPng (RIP next generation) is the IPv6 version of RIP. It is enabled per-interface, unlike IPv4 RIP.

! Enable IPv6 routing:

Router(config)# ipv6 unicast-routing

! Configure interfaces with IPv6 addresses:

Router(config)# interface fa0/0

Router(config-if)# ipv6 address 2001:DB8:1::1/64

Router(config-if)# ipv6 rip RIPNG enable   ! Enable RIPng on interface

Router(config-if)# no shutdown

Router(config)# interface fa0/1

Router(config-if)# ipv6 address 2001:DB8:2::1/64

Router(config-if)# ipv6 rip RIPNG enable

Router(config-if)# no shutdown

! Verify:

Router# show ipv6 route

Router# show ipv6 rip

# Module 3

Q1. Explain Cisco Wireless Technology.

Cisco wireless technology enables devices to connect to networks without physical cables using radio frequency (RF) signals.

Key Cisco wireless components:

- Cisco Aironet APs (Access Points): Provide wireless connectivity to clients

- Cisco WLC (Wireless LAN Controller): Centrally manages multiple APs

- CAPWAP: Protocol used between APs and WLC for management

- Cisco DNA Center: Centralized network management platform

Q2. List of IEEE Wireless Standards.

| Standard | Frequency | Max Speed | Range |
| --- | --- | --- | --- |
| 802.11a | 5 GHz | 54 Mbps | 35 meters |
| 802.11b | 2.4 GHz | 11 Mbps | 100 meters |
| 802.11g | 2.4 GHz | 54 Mbps | 100 meters |
| 802.11n (Wi-Fi 4) | 2.4/5 GHz | 600 Mbps | 250 meters |
| 802.11ac (Wi-Fi 5) | 5 GHz | 3.5 Gbps | 35 meters |
| 802.11ax (Wi-Fi 6) | 2.4/5/6 GHz | 9.6 Gbps | Indoor/Outdoor |

Q3. Explain Wireless Topologies.

- Ad Hoc (IBSS): Direct peer-to-peer connection without AP; e.g., laptop to laptop

- Infrastructure (BSS): All clients connect through a single Access Point

- Extended Service Set (ESS): Multiple APs connected to the same wired network; provides roaming

- Mesh Network: APs connect wirelessly to each other; used where wired backhaul is impractical

Q4. Explain Wireless Security Protocols and Encryption Method Types.

| Protocol | Year | Encryption | Security Level |
| --- | --- | --- | --- |
| WEP | 1999 | RC4 (64/128-bit) | Weak - easily cracked, deprecated |
| WPA | 2003 | TKIP + RC4 | Moderate - transitional standard |
| WPA2 | 2004 | AES-CCMP | Strong - current standard |
| WPA3 | 2018 | AES-GCMP + SAE | Strongest - latest standard |
| 802.1X/EAP | Enterprise | RADIUS server | Used in enterprise environments |

Q5. Example of DHCP Configuration.

DHCP (Dynamic Host Configuration Protocol) automatically assigns IP addresses and network configuration to clients.

! Configure DHCP server on router:

Router(config)# ip dhcp pool SALES

Router(config-dhcp)# network 192.168.10.0 255.255.255.0

Router(config-dhcp)# default-router 192.168.10.1

Router(config-dhcp)# dns-server 8.8.8.8

Router(config-dhcp)# lease 7            ! 7 days

! Exclude static addresses from DHCP pool:

Router(config)# ip dhcp excluded-address 192.168.10.1 192.168.10.10

! Verify:

Router# show ip dhcp binding

Router# show ip dhcp pool

Q6. What is ACL? Types of ACL and Example of Extended ACL.

ACL (Access Control List) is a set of rules used to filter network traffic based on specified criteria. Applied on router interfaces to permit or deny packets.

Types of ACL:

- Standard ACL (1-99, 1300-1999): Filters based on source IP address only

- Extended ACL (100-199, 2000-2699): Filters based on source IP, destination IP, protocol, and port

- Named ACL: Uses names instead of numbers; easier to manage

Extended ACL Example (deny FTP from PC to Server, permit rest):

Router(config)# ip access-list extended BLOCK_FTP

Router(config-ext-nacl)# deny tcp 192.168.1.0 0.0.0.255 host 10.0.0.5 eq 21

Router(config-ext-nacl)# permit ip any any

! Apply ACL to interface:

Router(config)# interface fa0/0

Router(config-if)# ip access-group BLOCK_FTP in

Q7. Example of Port Security in Switch.

Port security restricts input to an interface by limiting and identifying MAC addresses of the stations allowed to access the port.

! Configure port security on Fa0/1:

Switch(config)# interface fa0/1

Switch(config-if)# switchport mode access

Switch(config-if)# switchport port-security

Switch(config-if)# switchport port-security maximum 2    ! Max 2 MACs

Switch(config-if)# switchport port-security mac-address sticky  ! Auto-learn MACs

Switch(config-if)# switchport port-security violation restrict  ! Options: protect/restrict/shutdown

! Verify:

Switch# show port-security interface fa0/1

Q8. List of WAN Connections with Protocol.

| WAN Type | Protocol | Description |
| --- | --- | --- |
| Leased Line | HDLC, PPP | Dedicated point-to-point connection |
| Frame Relay | Frame Relay | Packet-switched WAN; uses VCs |
| DSL | PPPoE | Digital Subscriber Line over telephone |
| Cable | DOCSIS | Broadband over coaxial cable |
| MPLS | MPLS | Label-switched path; enterprise WAN |
| VPN | IPSec, GRE | Encrypted tunnel over public internet |
| Metro Ethernet | Ethernet | High-speed WAN using Ethernet |
| 4G/5G | Cellular | Mobile WAN connection |

Q9. Explain Frame-Relay and PPP.

Frame Relay:

Frame Relay is a packet-switched WAN protocol that operates at Layer 2. It uses Virtual Circuits (VCs) identified by DLCIs (Data Link Connection Identifiers).

- PVC (Permanent Virtual Circuit): Always active, pre-configured

- SVC (Switched Virtual Circuit): Established on demand

- LMI (Local Management Interface): Maintains status between DTE and DCE

PPP (Point-to-Point Protocol):

PPP is a Layer 2 WAN protocol used for direct connections between two routers. More feature-rich than HDLC.

- Supports authentication: PAP (unencrypted), CHAP (encrypted)

- Supports multiple network layer protocols

- Provides error detection and compression

! PPP with CHAP authentication:

Router(config)# interface serial 0/0

Router(config-if)# encapsulation ppp

Router(config-if)# ppp authentication chap

Q10. What is NAT? Explain with One Example.

NAT (Network Address Translation) translates private IP addresses to public IP addresses, allowing multiple devices on a LAN to access the internet using a single public IP.

Types of NAT:

- Static NAT: One-to-one mapping of private to public IP

- Dynamic NAT: Many-to-many using a pool of public IPs

- PAT (Overload/NAT Overload): Many-to-one; uses port numbers to distinguish sessions

PAT Example (most common):

! Define inside and outside interfaces:

Router(config)# interface fa0/0         ! LAN side

Router(config-if)# ip nat inside

Router(config)# interface fa0/1         ! WAN/Internet side

Router(config-if)# ip nat outside

! Create access list for inside hosts:

Router(config)# access-list 1 permit 192.168.1.0 0.0.0.255

! Apply NAT overload:

Router(config)# ip nat inside source list 1 interface fa0/1 overload

! Verify:

Router# show ip nat translations

Q11. What is HDLC? Which Command is Used to Show in Software?

HDLC (High-Level Data Link Control) is the default serial encapsulation protocol used on Cisco point-to-point serial links. It is a Layer 2 protocol.

Key features:

- Cisco's HDLC is proprietary (not compatible with other vendors)

- Lightweight with minimal overhead

- No authentication support

! HDLC is the default encapsulation; to verify:

Router(config)# interface serial 0/0

Router(config-if)# encapsulation hdlc   ! Explicitly set HDLC

! Show commands:

Router# show interfaces serial 0/0

Router# show ip interface brief

! Look for: Encapsulation HDLC

Q12. What is Encapsulation? Example of GRE Tunnel.

Encapsulation (in WAN context) refers to wrapping data from one protocol inside another protocol's frame/packet. This allows packets to travel across networks that may not natively support them.

GRE (Generic Routing Encapsulation) Tunnel:

GRE is a tunneling protocol that encapsulates a wide variety of network layer protocols inside IP tunnels. Used to create VPN-like point-to-point connections over the internet.

! GRE Tunnel Example: R1 (10.0.0.1) to R2 (10.0.0.2)

! On R1:

R1(config)# interface tunnel 0

R1(config-if)# ip address 172.16.0.1 255.255.255.0   ! Tunnel IP

R1(config-if)# tunnel source 10.0.0.1                ! R1's real IP

R1(config-if)# tunnel destination 10.0.0.2           ! R2's real IP

! On R2:

R2(config)# interface tunnel 0

R2(config-if)# ip address 172.16.0.2 255.255.255.0

R2(config-if)# tunnel source 10.0.0.2

R2(config-if)# tunnel destination 10.0.0.1

# Module 4

Q1. List of IP Services Types and Example of HSRP.

Common IP Services:

- DHCP: Dynamic IP assignment

- DNS: Domain name to IP resolution

- NAT/PAT: Address translation

- NTP: Network Time Protocol – synchronizes device clocks

- SNMP: Simple Network Management Protocol – monitoring

- Syslog: Logging and event tracking

- HSRP/VRRP/GLBP: First Hop Redundancy Protocols

HSRP (Hot Standby Router Protocol) Example:

HSRP provides gateway redundancy. Two routers share a virtual IP; one is Active, the other Standby.

! On Router 1 (Active):

R1(config)# interface fa0/0

R1(config-if)# ip address 192.168.1.2 255.255.255.0

R1(config-if)# standby 1 ip 192.168.1.1    ! Virtual IP

R1(config-if)# standby 1 priority 110      ! Higher = preferred Active

R1(config-if)# standby 1 preempt           ! Reclaim Active if higher priority

! On Router 2 (Standby):

R2(config)# interface fa0/0

R2(config-if)# ip address 192.168.1.3 255.255.255.0

R2(config-if)# standby 1 ip 192.168.1.1

R2(config-if)# standby 1 priority 100      ! Default = 100

! Verify:

Router# show standby brief

Q2. Example of Backup and Restore Router Managing IOS.

IOS management involves saving, backing up, and restoring the Cisco IOS image and configuration.

Backup Configuration:

! Save running config to startup config (NVRAM):

Router# copy running-config startup-config

! Backup config to TFTP server:

Router# copy running-config tftp

Address or name of remote host? 192.168.1.100

Destination filename? router-config-backup

Restore Configuration:

! Restore from TFTP server:

Router# copy tftp running-config

Address or name of remote host? 192.168.1.100

Source filename? router-config-backup

Backup IOS Image:

Router# copy flash tftp

Source filename? c2900-universalk9-mz.SPA.155-3.M4a.bin

Address or name of remote host? 192.168.1.100

Q3. Explain Security Threat.

Network security threats are attempts to compromise the confidentiality, integrity, or availability of network resources.

Types of Security Threats:

| Threat Type | Description | Example |
| --- | --- | --- |
| Reconnaissance | Information gathering about a target | Port scanning, ping sweep |
| Access Attack | Unauthorized access to resources | Password cracking, man-in-the-middle |
| DoS/DDoS | Overloading a system to deny service | SYN flood, ICMP flood |
| Malware | Malicious software | Virus, ransomware, trojan, worm |
| Social Engineering | Manipulating people | Phishing, vishing, pretexting |
| Data Exfiltration | Stealing sensitive data | SQL injection, insider threat |
| VLAN Hopping | Attacking switch VLANs | Double tagging, switch spoofing |

Q4. List of Basic Security of Password – Example with Apply Password in Router.

Cisco IOS provides multiple levels of password protection:

| Password Type | Command | Description |
| --- | --- | --- |
| Console password | line console 0 / password / login | Secures physical console access |
| VTY password | line vty 0 4 / password / login | Secures Telnet/SSH remote access |
| Enable password | enable password | Secures privileged exec (plaintext) |
| Enable secret | enable secret | Secures privileged exec (MD5 encrypted) |
| Service password-encryption | service password-encryption | Encrypts all plaintext passwords |

Complete Password Configuration Example:

Router(config)# enable secret Cisco@123        ! Encrypted enable password

Router(config)# line console 0

Router(config-line)# password Console@1

Router(config-line)# login

Router(config-line)# exec-timeout 5 0          ! Timeout after 5 min

Router(config)# line vty 0 4

Router(config-line)# password VTY@Pass1

Router(config-line)# login

Router(config-line)# transport input ssh        ! Allow only SSH

Router(config)# service password-encryption     ! Encrypt all passwords

! Secure unused ports:

Router(config)# no cdp run                      ! Disable CDP globally

Router(config)# banner motd # Unauthorized access is prohibited! #

Q5. Describe Threat Defense Technologies.

Modern networks use multiple layers of defense to protect against security threats:

| Technology | Description |
| --- | --- |
| Firewall | Filters traffic based on rules; stateful or stateless inspection |
| IPS/IDS | Intrusion Prevention/Detection System; monitors and blocks attacks |
| ACLs | Access Control Lists; filter traffic on router interfaces |
| 802.1X / AAA | Network access control using RADIUS/TACACS+ authentication |
| VPN | Encrypts data in transit over public networks (IPSec, SSL/TLS) |
| Antivirus/Antimalware | Endpoint protection against malicious software |
| Network Segmentation | VLANs and subnetting to contain security incidents |
| Cisco ASA / FTD | Cisco's dedicated firewall and intrusion prevention appliances |
| Syslog / SIEM | Centralized logging and event correlation for monitoring |
| Port Security | Restricts MAC addresses on switch ports to prevent rogue devices |

# Module 5

Q1. Explain How Automation Impacts Network Management.

Network automation uses software to automatically configure, manage, test, and operate network devices, replacing manual, repetitive tasks with programmatic approaches.

Impact of Automation:

- Speed: Configurations that took hours can be deployed in seconds across hundreds of devices

- Consistency: Eliminates human error; same configuration applied uniformly

- Scalability: Easily manage thousands of devices without adding staff

- Agility: Networks can respond to business needs faster

- Cost reduction: Fewer manual operations, less operational overhead

- Tools used: Ansible, Python (Netmiko, NAPALM), Cisco NSO, Puppet, Chef

Q2. Compare Traditional Network with Controller-Based Networking.

| Aspect | Traditional Networking | Controller-Based (SDN) |
| --- | --- | --- |
| Architecture | Distributed control plane on each device | Centralized control plane (controller) |
| Configuration | Manual CLI on each device | Centralized, programmatic via controller |
| Flexibility | Limited; hardware-bound | High; software-defined |
| Scalability | Difficult; device-by-device | Easy; controller manages all devices |
| Automation | Limited; scripts per device | Built-in; REST APIs |
| Troubleshooting | Per-device analysis | Centralized visibility |
| Cost | High OpEx (manual) | Lower OpEx (automated) |
| Examples | Standalone routers/switches | Cisco DNA Center, OpenFlow |

Q3. Explain Virtualization.

Virtualization is the creation of a virtual (software-based) version of something, such as servers, storage, networks, or operating systems, running on a physical host.

Types of Virtualization in Networking:

- Server Virtualization: Multiple VMs run on a single physical server (VMware, Hyper-V, KVM)

- Network Virtualization: Abstracting network resources; VLANs, VRFs, overlay networks

- NFV (Network Function Virtualization): Running network functions (firewall, router) as VMs instead of dedicated hardware

- SD-WAN: Virtualized WAN management across multiple transport types

Benefits:

- Resource optimization: Better hardware utilization

- Cost savings: Fewer physical devices needed

- Faster deployment: Spin up virtual instances quickly

- Disaster recovery: Easier backup and restoration of VMs

Q4. Describe Characteristics of REST-Based API.

REST (Representational State Transfer) is an architectural style for designing networked applications. REST APIs use standard HTTP methods to interact with resources.

Key Characteristics:

- Stateless: Each request is independent; server stores no client session state

- Client-Server: Clear separation between client (requester) and server (resource provider)

- Uniform Interface: Standard HTTP methods used: GET, POST, PUT, DELETE, PATCH

- Resource-Based: Everything is a resource identified by a URI (e.g., /api/v1/devices/1)

- Cacheable: Responses can be cached for performance

- Layered System: Intermediaries (proxies, gateways) can exist transparently

Data Format: REST APIs typically exchange data in JSON or XML format.

! Example REST API call (Python):

GET https://dna-center/dna/intent/api/v1/network-device

Headers: {Content-Type: application/json, Token: <auth_token>}

Response: JSON list of network devices

Q5. Explain DNA Center.

Cisco DNA (Digital Network Architecture) Center is Cisco's network management and automation platform. It provides a centralized controller for intent-based networking.

Key capabilities:

- Design: Create network hierarchy, define sites, IP pools, and network profiles

- Policy: Define and apply business intent as network policies

- Provision: Zero-touch provisioning and automated configuration deployment

- Assurance: AI/ML-powered monitoring, analytics, and troubleshooting

- Platform: Open REST APIs for integration with third-party tools

Benefits:

- Reduces operational complexity

- Faster service deployment

- AI-driven insights and proactive issue resolution

- Single pane of glass for network visibility

Q6. Explain SDN (Software-Defined Networking).

SDN (Software-Defined Networking) is an approach to network management that separates the control plane (decision-making) from the data plane (packet forwarding), centralizing control in a software controller.

SDN Architecture Layers:

- Application Layer: Network applications (firewalls, load balancers) communicate with the controller via Northbound APIs (REST)

- Control Layer: SDN Controller (e.g., OpenDaylight, Cisco APIC) makes routing/forwarding decisions

- Infrastructure Layer: Network devices (switches, routers) forward traffic based on instructions from controller via Southbound APIs (OpenFlow, NETCONF, RESTCONF)

| Feature | Traditional | SDN |
| --- | --- | --- |
| Control Plane | Distributed (on each device) | Centralized (controller) |
| Data Plane | On each device | On each device (unchanged) |
| Configuration | CLI per device | Programmatic via controller |
| Protocols | OSPF, EIGRP, STP | OpenFlow, NETCONF, RESTCONF |
| Vendors | Cisco, Juniper, etc. | OpenDaylight, Cisco ACI, VMware NSX |