What is System Bus?

A system bus is the primary communication pathway that connects the CPU, memory (RAM), and other hardware components on the motherboard. It transfers data, addresses, and control signals between these components.

The system bus consists of three sub-buses:

1. Data Bus — Carries the actual data being transferred between components. Its width (8, 16, 32, or 64 bits) determines how much data can be transferred at once.

2. Address Bus — Carries memory addresses, telling the system where data should be read from or written to. A 32-bit address bus can address up to 4 GB of memory.

3. Control Bus — Carries control signals from the CPU to coordinate and manage operations (read, write, interrupt signals, etc.).

What is Chipset and Types of Chipset?

A chipset is a group of integrated circuits (chips) on the motherboard that manages data flow between the CPU, RAM, storage, and peripheral devices. It acts as the "traffic controller" of the motherboard.

Types of Chipset:

1. Northbridge (MCH – Memory Controller Hub)

- Connects the CPU to high-speed components: RAM, AGP/PCIe (GPU), and the Southbridge.

- Handles memory-intensive, time-critical tasks.

- Located closer to the CPU on the motherboard.

- Modern CPUs (Intel Core i-series, AMD Ryzen) have absorbed Northbridge functions directly into the CPU die.

2. Southbridge (ICH – I/O Controller Hub)

- Handles slower, lower-priority peripherals: USB ports, SATA, audio, PCI slots, BIOS chip, and the real-time clock.

- Communicates with the CPU through the Northbridge.

Modern Single-Chip Chipsets (PCH – Platform Controller Hub)

- Intel and AMD now use a single chipset called PCH that replaces both Northbridge and Southbridge, since Northbridge functions moved into the CPU itself.

How Does the Northbridge Chipset Work?

The Northbridge sits between the CPU and the high-bandwidth components. Here's how it works:

- The CPU sends a request (e.g., read data from RAM) via the Front Side Bus (FSB).

- The Northbridge receives the request and checks whether it's for RAM or the GPU.

- If it's for RAM → it communicates via the memory bus to the RAM and returns the data to the CPU.

- If it's for the GPU → it routes the request through the AGP or PCIe bus to the graphics card.

- For lower-priority tasks (USB, audio, SATA), the Northbridge forwards the request to the Southbridge.

What is SMPS? And Its Purpose

### SMPS (Switched Mode Power Supply) is the power supply unit (PSU) inside a computer that converts AC (Alternating Current) from the wall outlet into the DC (Direct Current) voltages that computer components require.

Purpose of SMPS:

- Converts 220V/110V AC to usable DC voltages.

- Regulates and stabilizes voltage to protect components.

- Distributes power to all components via connectors (24-pin ATX, 8-pin CPU, SATA power, Molex, PCIe).

- Contains a cooling fan to dissipate heat.

Practical

- Open the computer case (remove side panel).

- Slide the SMPS into the PSU bay (usually top-rear or bottom-rear of the case).

- Secure it with four screws from the back of the case.

- Connect the 24-pin ATX connector to the motherboard.

- Connect the 4-pin or 8-pin EPS connector to the CPU power socket.

- Connect SATA power cables to hard drives and SSDs.

- Connect PCIe power cables to the GPU if needed.

- Connect Molex connectors to case fans or optical drives.

- Close the case and power on.

How to Check SMPS?

Method 1 – Paperclip Test (Without Motherboard)

- Unplug the SMPS from all components.

- Take the 24-pin ATX connector.

- Use a paperclip or wire and short pin 16 (PS_ON – green wire) to any black wire (ground).

- Plug the SMPS into the wall and switch it on.

- If the fan inside the SMPS spins → SMPS is working.

- Use a multimeter on the yellow (+12V), red (+5V), and orange (+3.3V) wires to verify correct voltages.

Method 2 – PSU Tester Tool

- Connect a dedicated PSU tester to the 24-pin connector.

- The tester displays voltage readings on an LCD screen.

- Check that all rails are within acceptable ranges (±5% tolerance).

Method 3 – Inside the System

- Power on the system and observe if all fans spin.

- If the system powers on but components fail → check individual rail voltages with a multimeter.

- If the system doesn't power on at all → suspect SMPS failure.

List Out the Types of Storage Devices

1. HDD (Hard Disk Drive) — Magnetic platters, slow but cheap, high capacity (500 GB – 20 TB).

2. SSD (Solid State Drive) — Flash memory, fast, no moving parts. Types: SATA SSD, NVMe M.2 SSD, PCIe SSD.

3. Optical Drives — CD, DVD, Blu-ray. Use laser to read/write data.

4. USB Flash Drive (Pen Drive) — Portable flash storage, plug-and-play via USB.

5. SD Card / Memory Card — Used in cameras, phones, embedded devices.

6. Magnetic Tape — Used for large-scale data archiving and backups in enterprises.

7. NAS (Network Attached Storage) — Storage accessible over a network.

8. Cloud Storage — Remote storage accessed via internet (Google Drive, OneDrive, etc.).

Describe the Working Process of Storage Devices

HDD:

- The spindle motor spins magnetic platters at 5400–7200 RPM.

- The read/write head (on the actuator arm) hovers nanometers above the platter.

- To write: the head magnetizes sectors of the platter.

- To read: the head detects magnetic polarity changes and converts them to electrical signals → binary data.

- Data is organized into tracks, sectors, and cylinders.

SSD:

- Uses NAND flash memory cells (no moving parts).

- Data is stored as electrical charges in floating-gate transistors.

- The SSD controller manages read/write operations, wear leveling, and error correction.

- Reading is nearly instant — the controller directly addresses the memory cell.

- Writing requires erasing a block before writing (hence wear leveling is important).

Do a practical to Remove storage devices and reinstall it and make a gpt disk.

Removing and Reinstalling:

- Power off the system and open the case.

- Disconnect the SATA data cable and SATA power cable from the drive.

- Unscrew the drive from the drive bay and remove it.

- Reinstall by reversing the steps — mount drive, connect power and data cables.

Making a GPT Disk (Windows):

- Press Win + X → select Disk Management

- Find the disk → right-click → Initialize Disk.

- Choose GPT (GUID Partition Table) → click OK.

Via CMD (Diskpart):

diskpart

list disk

select disk 1

clean

convert gpt

What is SATA

SATA (Serial Advanced Technology Attachment) is a standard interface used to connect storage devices (HDD, SSD, optical drives) to the motherboard. It replaced the older PATA (Parallel ATA) standard.

Describe the Working of SATA

- The SATA controller (on the motherboard/chipset) manages communication between the CPU and the storage device.

- Data is sent serially (one bit at a time) over a 7-pin data cable — unlike PATA which sent data in parallel over 40/80 wires.

- When the OS requests a file, the SATA controller sends a command to the drive.

- The drive retrieves the data and sends it back through the SATA data cable to the controller, then to the CPU/RAM.

- SATA uses AHCI (Advanced Host Controller Interface) protocol for features like NCQ (Native Command Queuing) and hot-swapping.

Do a practical to install SATA.

1. Power off the computer and open the case.

2.Mount the HDD/SSD into the drive bay and screw it in.

3. Connect the 7-pin SATA data cable from the drive to a SATA port on the motherboard.

4.Connect the SATA power cable (15-pin, L-shaped) from the SMPS to the drive.

5.Power on the system.

6.Go to Disk Management (Win + X) to initialize and format the new drive.

7.In BIOS, verify the drive is detected under the SATA configuration section.

What is SCSI Storage and Types of SCSI?

SCSI (Small Computer System Interface) is a set of standards for connecting and transferring data between computers and peripheral devices (hard drives, tape drives, scanners, printers). It allows multiple devices to be connected in a chain on a single bus.

SCSI is primarily used in servers and enterprise environments due to its high speed and ability to connect multiple devices.

Types of SCSI:

| Type | Bus Width | Max Speed | Max Devices |
| --- | --- | --- | --- |
| SCSI-1 | 8-bit | 5 MB/s | 8 |
| SCSI-2 (Fast SCSI) | 8-bit | 10 MB/s | 8 |
| Wide SCSI | 16-bit | 20 MB/s | 16 |
| Ultra SCSI | 8-bit | 20 MB/s | 8 |
| Ultra Wide SCSI | 16-bit | 40 MB/s | 16 |
| Ultra2 SCSI | 8-bit | 40 MB/s | 8 |
| Ultra160 SCSI | 16-bit | 160 MB/s | 16 |
| Ultra320 SCSI | 16-bit | 320 MB/s | 16 |
| iSCSI | N/A (network) | Depends on network | Many |
| SAS (Serial Attached SCSI) | Serial | Up to 12 Gbps | 65,535 |

What is I/O Ports?

I/O Ports (Input/Output Ports) are physical connectors or interfaces on a computer that allow it to communicate with external devices and peripherals. They serve as the bridge between the internal components of the computer and the outside world.

I/O ports can be used for input (receiving data), output (sending data), or both.

List out the I/O ports available Do a practical to identify the I/O ports

Common I/O Ports:

USB Ports (Universal Serial Bus)

- USB 2.0 — Up to 480 Mbps (black connector)

- USB 3.0 / 3.1 Gen 1 — Up to 5 Gbps (blue connector)

- USB 3.1 Gen 2 — Up to 10 Gbps

- USB 3.2 / USB4 — Up to 20–40 Gbps

- USB Type-C — Reversible connector, supports USB, Thunderbolt, DisplayPort

Video/Display Ports

- VGA (D-Sub 15-pin) — Analog, older monitors

- DVI (Digital Visual Interface) — Digital/analog, older displays

- HDMI — Digital audio + video, most common today

- DisplayPort / Mini DisplayPort — High refresh rate, multi-monitor support

- Thunderbolt (1/2/3/4) — High-speed data + display (uses USB-C connector from TB3 onward)

Audio Ports

- 3.5mm Audio Jack — Headphone (green), Microphone (pink), Line-in (blue)

- S/PDIF (Optical/Coaxial) — Digital audio output

Network Ports

- RJ-45 (Ethernet LAN port) — Wired network connection

Legacy / Older Ports

- PS/2 — Keyboard (purple) and Mouse (green) — circular 6-pin connectors

- Serial Port (COM Port, DB-9) — Used for older serial devices

- Parallel Port (LPT, DB-25) — Used for older printers

- RS-232 — Serial communication for legacy devices

Storage Interface Ports (External)

- eSATA — External SATA for external hard drives

- SD Card Reader — Reads SD/microSD cards

Practical – How to Identify I/O Ports:

- Look at the rear panel (I/O shield) of the desktop or the sides of a laptop.

- USB ports are rectangular (Type-A) or oval (Type-C); color indicates speed (black = USB 2.0, blue = USB 3.0).

- The RJ-45 port looks like a wide phone jack.

- Audio ports are 3.5mm round jacks, color-coded (green = headphone, pink = mic).

- HDMI is a trapezoidal 19-pin port; VGA is a blue trapezoid with 15 pins in 3 rows.

- PS/2 ports are small, round, and color-coded (purple = keyboard, green = mouse).

- Use Device Manager (Windows) or lspci / lsusb (Linux) to identify ports programmatically.