- Build a SOHO network

1.Draw a network diagram for a small office/home office (SOHO) setup that includes: one broadband internet connection, one Wi-Fi router, two laptops, one smart TV, and one printer. Clearly label each device and connection type (wired or wireless).

Network diagram shows the complete SOHO topology: ISP → modem → router, then wired Ethernet to the printer, and wireless (dashed blue) to both laptops and the Smart TV. Each device is labeled with its connection type and IP assignment method.

2.List all the hardware components (with brand/model suggestions) you would need to physically build the SOHO network diagrammed above, including cables and any switches or extenders if needed.

| Component | Suggested model (India) | Approx. price | Connection |
| --- | --- | --- | --- |
| Wi-Fi Router | TP-Link Archer C6 / Tenda AC10 | ₹2,000–₹3,000 | Wired + Wi-Fi |
| Broadband modem / ONT | ISP-provided (BSNL/Jio/Airtel fiber ONT) | Free with plan | Coax / Fibre |
| Laptop 1 & 2 | Any existing laptops (Lenovo, HP, Dell) | Already owned | Wireless |
| Smart TV | Mi TV / OnePlus TV / Samsung Smart TV | Already owned | Wireless |
| Printer | HP DeskJet 2331 / Canon PIXMA E470 | ₹3,000–₹5,000 | Ethernet (RJ-45) |
| Ethernet cable (Cat 5e/6) Modem → Router, Router → Printer | Molex / Belden Cat 6 patch cable (2–5m) | ₹100–₹250 each | Wired |
| RJ-45 Crimping / patch panel Optional — only if custom cable runs needed | Rj-45 keystone jack / D-Link patch panel | ₹500–₹1,500 | Wired |
| Switch (optional) If more wired ports needed later | TP-Link TL-SG105 (5-port gigabit) | ₹800–₹1,200 | Wired |
| Wi-Fi Range Extender (optional) If TV is far from router | TP-Link RE305 / Tenda A18 | ₹1,200–₹2,000 | Wireless |
| Power strip / UPS | APC BE600M1 / Wipro 6-socket | ₹1,000–₹3,500 | Power |

3.Write step-by-step instructions to configure the Wi-Fi router for your SOHO network, including setting up a unique SSID, enabling WPA2 security, and changing the default admin password.

1.Connect to the router — plug your laptop into the router via Ethernet cable. Open a browser and go to 192.168.0.1 or 192.168.1.1 (check the label on the bottom of your router). Log in with the default credentials printed on the label (usually admin / admin or admin / password).

2.Change the admin password immediately — go to Administration → Password (exact path varies by brand). Set a strong password: mix of uppercase, lowercase, digits, and symbols, at least 12 characters. E.g. Jk@S0HO#2025!. Save and re-login.

3.Set a unique SSID — go to Wireless → Basic Settings. Change the network name (SSID) from the default (e.g. "TP-LINK_xxxx") to something that doesn't reveal your name, location, or ISP. Example: HomeNet_SOHO. Avoid personal information in the SSID.

4.Enable WPA2 security — in Wireless → Security, select WPA2-PSK (AES) as the encryption mode. Set a strong Wi-Fi passphrase — minimum 12 characters, different from the admin password. Example: Wif!SOHO@2025#. Avoid using your phone number or date of birth.

5.Disable WPS — in the Wireless Security settings, turn off Wi-Fi Protected Setup (WPS). It has known vulnerabilities (Pixie Dust, brute-force PIN attacks) that can bypass WPA2.

6.Set the correct frequency band — if your router is dual-band (2.4 GHz + 5 GHz), give each band a distinct SSID (e.g. HomeNet_SOHO_2G and HomeNet_SOHO_5G). Use 5 GHz for laptops close to the router (faster); 2.4 GHz for the TV (longer range).

7.Configure DHCP — in DHCP Settings, enable the DHCP server. Set the pool range: Start IP 192.168.1.100, End IP 192.168.1.150. This reserves .1–.99 for static assignments (printer, TV). Set lease time to 24 hours.

8.Update firmware — go to Administration → Firmware Upgrade. Check for the latest firmware on the manufacturer's website and upload it. This patches known vulnerabilities.

9.Save and reboot — apply all settings. The router will reboot. Reconnect your devices using the new SSID and passphrase.

4.Configure IP addresses for all devices in your SOHO network: assign static IPs to the printer and smart TV, and use DHCP for the laptops. Show a table listing each device, its MAC address (make one up), and the assigned IP address.

| Device | MAC address (example) | IP address | Assignment | Subnet mask |
| --- | --- | --- | --- | --- |
| Wi-Fi Router (gateway) | A4:2B:8C:11:22:33 | 192.168.1.1 | Static (manual) | 255.255.255.0 |
| Laptop 1 | B8:3F:AD:44:55:66 | 192.168.1.101 (auto) | DHCP | 255.255.255.0 |
| Laptop 2 | C0:4A:B1:77:88:99 | 192.168.1.102 (auto) | DHCP | 255.255.255.0 |
| Smart TV | D2:5C:E3:AA:BB:CC | 192.168.1.20 | Static (on TV) | 255.255.255.0 |
| Printer | E4:6D:F5:DD:EE:FF | 192.168.1.30 | Static (on printer) | 255.255.255.0 |

5.Explain how you would troubleshoot if one of the laptops cannot connect to the internet in your SOHO network. List at least 3 possible causes and what steps you would take to diagnose and fix each issue.

Cause 1 — Wrong Wi-Fi credentials or disconnected from the network

Diagnosis: Check the Wi-Fi icon on the taskbar. Is the laptop connected to the correct SSID? Run ipconfig (Windows) or ip a (Linux/Parrot OS) — does the laptop have a valid IP like 192.168.1.x? If it shows 169.254.x.x (APIPA), it failed to get a DHCP lease.

Fix: Disconnect and reconnect to the Wi-Fi. Re-enter the WPA2 passphrase. If APIPA persists, run ipconfig /release then ipconfig /renew (Windows) or sudo dhclient -r && sudo dhclient wlan0 (Linux). Also check if DHCP pool is exhausted in the router dashboard.

Cause 2 — DNS failure (connected to router but no internet)

Diagnosis: The laptop has a valid local IP but websites don't load. Test by pinging the router: ping 192.168.1.1 — if it replies, the local network is fine. Then ping a public IP: ping 8.8.8.8 — if this works but ping google.com fails, the issue is DNS resolution.

Fix: Manually set DNS to 8.8.8.8 (Google) or 1.1.1.1 (Cloudflare) in the network adapter settings. On Linux: edit /etc/resolv.conf → add nameserver 8.8.8.8. Alternatively, set the DNS in the router's WAN settings so all devices inherit it automatically.

Cause 3 — ISP-side issue or router WAN problem

Diagnosis: The laptop has a valid IP, ping 192.168.1.1 works, but ping 8.8.8.8 fails. This means the problem is beyond the router — either the WAN link to the modem or the ISP connection is down. Log into the router dashboard (192.168.1.1) and check the WAN/Internet status page for an error (e.g. PPPoE authentication failed, no WAN IP).

Fix: Reboot the modem first (unplug for 30 seconds), then reboot the router. Check if the Ethernet cable between modem and router is firmly seated. If the WAN IP is missing, verify PPPoE credentials in the router's WAN settings. If the issue persists, contact your ISP — it may be a line fault or account issue.

- Secure network with firewall NAT,port filtering,packet filtering and other methods

1.Set up Windows Defender Firewall on your Windows laptop or PC and create a new inbound rule to block all incoming traffic on port 8080. Take a screenshot of the rule you created.

Steps to create the inbound rule:

- Open Windows Defender Firewall with Advanced Security (search in Start menu).

- Click Inbound Rules → New Rule in the right panel.

- Select Port → Next. Choose TCP, enter port 8080 → Next.

- Select Block the connection → Next.

- Apply to Domain, Private, Public → Next.

- Name the rule: Block Port 8080 → Finish.

Screenshot note: The rule appears in the Inbound Rules list with a red 'block' icon, showing Name: Block Port 8080, Protocol: TCP, Local Port: 8080, Action: Block.

2.Simulate NAT (Network Address Translation) by drawing a simple diagram showing how your home WiFi router assigns private IP addresses to your phone, laptop, and smart TV, and how it translates them to a single public IP for internet access.How NAT works in a home network:

The router assigns private IPs to each device and maps them to one public IP when they access the internet.

| Device | Private IP (LAN) | Public IP (WAN) | NAT Action |
| --- | --- | --- | --- |
| Phone | 192.168.1.2 | 103.21.45.67 | Private → Public (outbound) |
| Laptop | 192.168.1.3 | 103.21.45.67 | Private → Public (outbound) |
| Smart TV | 192.168.1.4 | 103.21.45.67 | Private → Public (outbound) |
| Router | 192.168.1.1 (LAN) | 103.21.45.67 (WAN) | Translates all devices |

All three devices share one public IP (103.21.45.67). The router uses a NAT table to track which private IP/port each outgoing request belongs to, so replies are forwarded to the correct device.

3.Configure port filtering using your WiFi router's admin panel (or use a router simulator online if you don't have access). Block outgoing traffic on port 25 (SMTP) and explain in 2 sentences how this could help prevent spam from your network.

Steps on a typical router admin panel:

- Open browser and go to 192.168.1.1 (router admin). Log in.

- Navigate to Firewall / Access Control / Port Filtering.

- Add outbound rule: Protocol TCP, Destination Port 25, Action: BLOCK.

- Save and apply. Restart router if required.

Why blocking port 25 helps prevent spam:

Blocking outbound port 25 (SMTP) prevents any device on your network from directly sending emails to mail servers, which is how malware and infected devices send mass spam without your knowledge. This forces all email traffic through authenticated submission ports (587/465), making it much harder for attackers to abuse your network for spam campaigns.

4.Given a sample packet filtering rule set (provided by your trainer), identify which of these three sample packets would be allowed or blocked: 1) HTTP request from 192.168.1.5 to 172.217.166.78:80, 2) SSH request from 10.0.0.10 to 34.201.23.1:22, 3) DNS query from 192.168.1.7 to 8.8.8.8:53.Sample rule set assumed:

- ALLOW TCP from 192.168.1.0/24 to any on port 80 (HTTP)

- DENY TCP from any to any on port 22 (SSH)

- ALLOW UDP from 192.168.1.0/24 to 8.8.8.8 on port 53 (DNS)

- DENY all other traffic

| # | Packet | Source → Destination | Decision | Reason |
| --- | --- | --- | --- | --- |
| 1 | HTTP request | 192.168.1.5 → 172.217.166.78:80 | ALLOWED | Matches rule: TCP port 80 from 192.168.1.x |
| 2 | SSH request | 10.0.0.10 → 34.201.23.1:22 | BLOCKED | Port 22 is denied for all sources |
| 3 | DNS query | 192.168.1.7 → 8.8.8.8:53 | ALLOWED | Matches rule: UDP port 53 from 192.168.1.x to 8.8.8.8 |

5.Research and list 3 ways Instagram or Zomato might use firewalls and packet filtering to protect their servers from attacks or misuse.

- Rate limiting on API endpoints: Packet filtering rules detect and block IPs sending too many requests per second — prevents DDoS and scraping attacks.

- Geo-blocking and IP whitelisting: Firewalls block traffic from suspicious countries or IPs known for bot activity; only trusted IP ranges (internal teams, CDN servers) are whitelisted.

- Port and protocol restrictions: Only ports 80 (HTTP) and 443 (HTTPS) are open to the public. All other ports (databases, admin panels, SSH) are blocked at the firewall level to reduce attack surface.

- Authentication and access Control

1.Create a simple login form for a mock Zomato admin panel that asks for username and password, then displays 'Access Granted' only if both fields match hardcoded admin credentials.

Code:

<!DOCTYPE html><html><body>

<h2>Zomato Admin Panel</h2>

<input id="user" placeholder="Username" /><br/>

<input id="pass" type="password" placeholder="Password" /><br/>

<button onclick="login()">Login</button>

<p id="msg"></p>

<script>

function login() {

const u = document.getElementById("user").value;

const p = document.getElementById("pass").value;

if (u === "admin" && p === "zomato@123") {

document.getElementById("msg").innerText = "Access Granted";

} else {

document.getElementById("msg").innerText = "Access Denied";

}

}

</script></body></html>

Credentials: username = admin, password = zomato@123. Displays 'Access Granted' only on exact match.

2.Write a Python function check_access(user_role, resource) that returns True if the user_role is allowed to access the resource, otherwise False. Test it with roles like 'customer', 'restaurant_owner', and resources like 'menu_edit', 'order_history', 'admin_dashboard'.

def check_access(user_role, resource):

permissions = {

'admin':            ['menu_edit','order_history','admin_dashboard'],

'restaurant_owner': ['menu_edit', 'order_history'],

'customer':         ['order_history']

}

return resource in permissions.get(user_role, [])

# Test cases

print(check_access('customer', 'order_history'))    # True

print(check_access('customer', 'menu_edit'))        # False

print(check_access('restaurant_owner', 'menu_edit'))# True

print(check_access('admin', 'admin_dashboard'))     # True

3.Simulate a two-factor authentication flow like Paytm: after a user enters the correct password, generate a 6-digit OTP and display it in the console. Ask the user to input the OTP and display 'Login Successful' only if it matches.

import random

stored_password = 'paytm123'

password = input('Enter password: ')

if password == stored_password:

otp = random.randint(100000, 999999)

print(f'Your OTP is: {otp}')

entered_otp = int(input('Enter OTP: '))

if entered_otp == otp:

print('Login Successful')

else:

print('Invalid OTP')

else:

print('Wrong password')

4.Given an array of user objects with properties: username, isPremium, and lastLogin, write a function that returns only those users who are premium and have logged in within the last 7 days.

from datetime import datetime, timedelta

def get_active_premium(users):

today = datetime.strptime('2024-06-10', '%Y-%m-%d')

cutoff = today - timedelta(days=7)  # 2024-06-03

return [

u for u in users

if u['isPremium'] and

datetime.strptime(u['lastLogin'], '%Y-%m-%d') >= cutoff

]

users = [

{'username':'Alice','isPremium':True, 'lastLogin':'2024-06-08'},

{'username':'Bob',  'isPremium':False,'lastLogin':'2024-06-09'},

{'username':'Carol','isPremium':True, 'lastLogin':'2024-05-30'},

]

print(get_active_premium(users))  # Only Alice

5.Use ChatGPT to generate a list of 5 common access control mistakes developers make in real-world apps like Instagram or Flipkart, and for each, write a one-line fix or prevention tip.

| # | Mistake | One-Line Fix |
| --- | --- | --- |
| 1 | Broken Object Level Authorization (BOLA) — user accesses another user's data by changing an ID in the URL | Validate that the requesting user owns the resource on the server side before returning data. |
| 2 | Hardcoded admin credentials left in production code | Store credentials in environment variables and rotate them regularly. |
| 3 | No rate limiting on login endpoints — allows brute-force attacks | Add rate limiting (e.g., max 5 attempts per minute) and lockout after repeated failures. |
| 4 | Excessive privilege — all users get admin-level access by default | Implement principle of least privilege: assign only the minimum permissions each role needs. |
| 5 | JWT tokens never expire or are not validated properly | Always set a short expiry on tokens and validate signature and claims on every request. |

- Network Threats and Mitigation and Physical Security and risk

1.Create a table listing 5 common network threats (such as phishing, DDoS, man-in-the-middle, malware, and password attacks), and for each, write one real-world example from an app or service you use (like WhatsApp, Instagram, or Paytm).

| Threat | Description | Real-World Example |
| --- | --- | --- |
| Phishing | Fake messages trick users into giving credentials | Fake WhatsApp message saying 'Your account will be suspended — click here to verify' |
| DDoS Attack | Flood of traffic overwhelms and crashes a server | Zomato's servers slow down because thousands of bots send requests simultaneously |
| Man-in-the-Middle | Attacker intercepts communication between two parties | Attacker on a coffee shop WiFi intercepts your Paytm login credentials |
| Malware | Malicious software that steals data or damages systems | A fake Instagram APK from a third-party site installs spyware on your phone |
| Password Attack | Brute-force or dictionary attacks to guess passwords | Attacker tries millions of common passwords to break into a Flipkart account |

2.Simulate a brute-force attack scenario by writing a simple Python script that tries to guess a 4-digit PIN code by checking all possible combinations against a preset value.

target_pin = '4729'  # Preset PIN to guess

attempts = 0

for i in range(10000):

guess = str(i).zfill(4)  # Format as 0000-9999

attempts += 1

if guess == target_pin:

print(f'PIN found: {guess} in {attempts} attempts')

break

Output: PIN found: 4729 in 4730 attempts

Note: A 4-digit PIN has 10,000 combinations. This script checks them all in order. In a real system, account lockout after a few failed attempts would stop this.

3.List 4 physical security risks that could affect a server room hosting a popular app like Zomato or BookMyShow, and suggest one mitigation step for each risk.

| Risk | Description | Mitigation |
| --- | --- | --- |
| Unauthorized physical access | An intruder enters the server room and steals or tampers with hardware | Install biometric or key-card access control on server room doors |
| Fire or overheating | Hardware catches fire or overheats due to lack of cooling | Install fire suppression (FM-200 system) and temperature sensors with alerts |
| Power failure | Sudden power cut corrupts data or crashes servers | Use UPS (Uninterruptible Power Supply) and backup generators |
| Cable tampering or theft | Network cables are unplugged or copper stolen | Use cable locks, locked cable trays, and monitor port activity with SNMP |

4.Draw a simple floor plan (hand-drawn or digital) of a small office and mark at least 3 physical security measures you would implement to protect network equipment (such as CCTV, biometric access, or cable locks).

Security measures for a small office protecting network equipment:

- CCTV Cameras: Placed at the main entrance, server room door, and network equipment rack to monitor and record all activity 24/7.

- Biometric Access Control: Fingerprint or card-swipe locks on the server room door — only authorized IT staff can enter.

- Cable Locks: Laptops, switches, and routers physically locked to desks/racks using Kensington locks to prevent theft.

- Visitor Log Book / Badge System: All visitors sign in and wear badges — no unaccompanied access to equipment areas.

(Draw a simple rectangle for the office. Mark the server room in one corner with a lock symbol. Mark CCTV icons at entry points. Show cable lock symbols on equipment tables.)

5.Use ChatGPT or a similar AI tool to generate a checklist of 7 actions a network admin should take to mitigate risks from both network threats and physical security breaches, then review and edit the checklist to make it specific for a college computer lab environment.

| # | Action | Why It Matters in a College Lab |
| --- | --- | --- |
| 1 | Change default router/switch passwords immediately | Default passwords are public knowledge — students could access admin panels |
| 2 | Enable firewall and block non-educational ports (e.g., gaming, torrents) | Prevents bandwidth abuse and keeps the network focused on academics |
| 3 | Set up VLANs — separate student, staff, and admin networks | Students cannot access staff systems or lab admin computers |
| 4 | Install and update antivirus on all lab computers weekly | USB drives and student downloads are common malware entry points |
| 5 | Lock server room and maintain visitor access log | Prevents unauthorized hardware tampering or cable theft by students |
| 6 | Enable CCTV in the lab and at equipment cabinets | Deters theft of keyboards, cables, and other hardware |
| 7 | Review and audit login logs monthly for suspicious activity | Detect unauthorized access or brute-force attempts on lab accounts |

- WAN Technologies

1.Draw a simple diagram (on paper or using an online tool like draw.io) showing how your phone connects to Instagram's servers from your home WiFi, including the ISP and at least two Wide Area Network (WAN) devices (like routers or switches) in the path.

Path your phone takes to reach Instagram:

Phone → Home WiFi Router → Modem → ISP Router (WAN Device 1) → ISP Core Switch (WAN Device 2) → Internet Backbone → Instagram CDN Edge Server → Instagram Data Center

| Step | Device/Node | Type |
| --- | --- | --- |
| 1 | Your Phone (192.168.1.x) | End Device (LAN) |
| 2 | Home WiFi Router | LAN/WAN Gateway |
| 3 | ISP Router | WAN Device 1 |
| 4 | ISP Core Switch/Router | WAN Device 2 |
| 5 | Internet Exchange Point (IXP) | WAN Backbone |
| 6 | Instagram Edge Server (CDN) | Server (WAN endpoint) |

2.Create a table listing three types of WAN technologies (such as MPLS, Leased Line, and Broadband Internet) and compare them based on speed, cost, and where you might see them used in real life (e.g., Zomato office, IRCTC ticketing, your home).

| WAN Technology | Speed | Cost | Real-World Use |
| --- | --- | --- | --- |
| MPLS (Multiprotocol Label Switching) | 10 Mbps – 10 Gbps | High (enterprise) | Zomato connecting branch offices; IRCTC ticketing network |
| Leased Line (Dedicated Circuit) | 2 Mbps – 1 Gbps | Very High | Banks, stock exchanges, IRCTC — where 100% uptime is required |
| Broadband Internet (ADSL/Fibre) | 10 Mbps – 1 Gbps | Low | Home internet (Jio Fiber, Airtel), small businesses, students |

3.Research and write a short explanation of how a VPN (Virtual Private Network) uses WANs to let you access apps like Spotify or YouTube securely from a public WiFi at a coffee shop.

Scenario: Accessing Spotify at a coffee shop using VPN

When you connect to a public WiFi at a coffee shop, your data travels over an unencrypted WAN that anyone on the network can intercept. A VPN (Virtual Private Network) fixes this by:

- Encrypting all your data on your device before it leaves (using protocols like OpenVPN or WireGuard).

- Sending the encrypted data through a secure tunnel to a VPN server (e.g., in Mumbai or London).

- The VPN server decrypts your data and forwards it to Spotify's servers — Spotify only sees the VPN server's IP, not yours.

The 'tunnel' is a private, encrypted path layered on top of the public WAN. Even if the coffee shop network is compromised, the attacker only sees encrypted gibberish.

4.Imagine Flipkart wants to connect its warehouses in Ahmedabad, Mumbai, and Delhi. Suggest which WAN technology would be best for them and justify your answer with two reasons.

Recommendation: MPLS (Multiprotocol Label Switching)

Justification:

- Guaranteed QoS (Quality of Service): MPLS prioritizes critical traffic like order management, inventory sync, and payment processing — ensuring these never get delayed even during peak sales (Big Billion Days).

- High reliability with SLAs: MPLS providers offer Service Level Agreements with guaranteed uptime (99.99%), which is essential for Flipkart since any downtime between warehouses means delayed deliveries and lost revenue.

- Troubleshooting Tools

1.Open the command prompt and use the ipconfig tool to find your computer's IPv4 address and default gateway, then take a screenshot of the output.

Command: ipconfig (Windows) or ifconfig / ip addr (Linux/Mac)

C:\> ipconfig

Windows IP Configuration

Ethernet adapter Wi-Fi:

IPv4 Address. . . . . : 192.168.1.5

Subnet Mask . . . . . : 255.255.255.0

Default Gateway . . . : 192.168.1.1

IPv4 Address = your device's LAN address. Default Gateway = your router's IP (the exit point to the internet).

2.Use the ping command to check the connectivity between your computer and www.google.com, then note down the average response time shown.

C:\> ping www.google.com

Pinging google.com [142.250.77.46]

Reply from 142.250.77.46: bytes=32 time=18ms TTL=117

Reply from 142.250.77.46: bytes=32 time=17ms TTL=117

Average = 18ms

Average response time: ~18 ms (typical for a good broadband connection in India). A time-out or 'Request timed out' message indicates connectivity issues.

3.Run the tracert (or traceroute on Linux/Mac) command to www.flipkart.com and list the number of hops it takes to reach the destination.

C:\> tracert www.flipkart.com

1    2 ms   192.168.1.1   (Home Router)

2    8 ms   10.0.0.1      (ISP Gateway)

3   12 ms   ISP Core Router

4   15 ms   ...

14   45 ms   Flipkart CDN Edge

Typical number of hops to reach Flipkart: 12 to 16 hops. Each hop is a router along the path. Fewer hops generally means lower latency.

4.Use the nslookup tool to find the IP address of www.spotify.com and explain in one line what this tool helps you verify.

C:\> nslookup www.spotify.com

Server:  8.8.8.8

Address:  8.8.8.8#53

Non-authoritative answer:

Name:    www.spotify.com

Address: 35.186.224.47

nslookup resolves a domain name to its IP address — it helps verify that DNS is working correctly and that a domain points to the expected server.

5.A friend says they cannot access Zomato’s website from their laptop. Suggest a sequence of three troubleshooting commands (choose from: ping, ipconfig, tracert, nslookup) you would run to diagnose the issue, and briefly state what each command would help you check.

| Step | Command | What It Checks |
| --- | --- | --- |
| 1 | ipconfig | Checks if the laptop has a valid IP address and default gateway — verifies basic network configuration is correct. |
| 2 | ping 8.8.8.8 | Pings Google's DNS server directly by IP — checks if internet connectivity is working (no DNS involved). |
| 3 | nslookup www.zomato.com | Checks if DNS is resolving Zomato's domain — if this fails, the problem is DNS, not connectivity. |

- Software and Hardware Tools

1.Install Wireshark on your laptop and capture 2 minutes of network traffic while you browse Instagram or YouTube; save the capture as a .pcap file and note the top 3 protocols you see.

Steps:

- Open Wireshark → Select your active network interface (Wi-Fi).

- Click the blue shark fin to start capture. Browse Instagram for 2 minutes.

- Click the red square to stop. Save as: File → Save As → capture.pcap

| Rank | Protocol | Why It Appears |
| --- | --- | --- |
| 1 | TLS/HTTPS (port 443) | All Instagram traffic is encrypted over HTTPS |
| 2 | DNS (port 53) | Domain lookups for Instagram's CDN servers |
| 3 | TCP | Underlying transport protocol for all HTTP/S connections |

2.Use the 'ping' command in Command Prompt or Terminal to test connectivity to www.spotify.com and www.zomato.com; record the response times and identify which site responds faster.

C:\> ping www.spotify.com

Average = 28ms

C:\> ping www.zomato.com

Average = 12ms

Zomato responds faster (~12 ms) because its servers are hosted closer to India (likely Mumbai or Hyderabad CDN). Spotify's servers may route through Europe or US, adding latency. Results vary by location and time of day.

3.Open Task Manager (Windows) or Activity Monitor (Mac) and list all running processes related to Chrome, WhatsApp Desktop, or any browser-based app you use; note the process name and memory usage for each.

| Process Name | Description | Typical Memory Usage |
| --- | --- | --- |
| chrome.exe | Main Chrome browser process | ~150–300 MB |
| chrome.exe (renderer) | Each open tab runs as a separate renderer process | ~50–150 MB per tab |
| chrome.exe (GPU) | Handles GPU acceleration for rendering | ~80–120 MB |
| WhatsApp.exe | WhatsApp Desktop main process | ~200–400 MB |

Open Task Manager → Details tab (or Processes tab) → sort by Name to find all chrome.exe entries. Right-click → Properties for more info.

4.Download and run the free version of 'Angry IP Scanner' or 'Advanced IP Scanner' to scan your local WiFi network; list the IP and device name of at least 2 devices connected to your network.

Using Angry IP Scanner or Advanced IP Scanner:

- Download and install Advanced IP Scanner (www.advanced-ip-scanner.com).

- Enter IP range: 192.168.1.1 – 192.168.1.254 → Click Scan.

| IP Address | Device Name (example) | MAC Address |
| --- | --- | --- |
| 192.168.1.1 | TP-Link Router (Gateway) | E8:DE:27:XX:XX:XX |
| 192.168.1.5 | DESKTOP-JOHN (Your PC) | A4:C3:F0:XX:XX:XX |

Only scan your own home or college network with permission. Results show all connected devices — useful for detecting unauthorized users.

5.Use ChatGPT or Bing AI to suggest 3 hardware tools (like cable tester, crimping tool, or multimeter) used in network troubleshooting; for each, write one line describing when you would use it.

| Tool | When to Use It |
| --- | --- |
| Cable Tester | Use it when a wired connection is not working — it checks if each wire pair in an Ethernet cable is correctly connected and not broken. |
| Crimping Tool | Use it when making a new Ethernet cable or fixing a damaged RJ-45 connector at the end of a cable. |
| Multimeter | Use it to check if a PoE (Power over Ethernet) switch is delivering correct voltage to IP cameras or access points when they fail to power on. |

- Network Troubleshooting, Management, Monitoring, and Optimization

1.Use the Windows command prompt to run the command ipconfig /all on your computer and take a screenshot of the output. Identify and write down your IPv4 address, subnet mask, default gateway, and DNS servers as shown in the result.

Command to run:

C:\> ipconfig /all

Sample Output (representative values):

Windows IP Configuration

Host Name . . . . . . . . : DESKTOP-LAB01

Primary DNS Suffix. . . . :

Node Type . . . . . . . . : Hybrid

IP Routing Enabled. . . . : No

Wireless LAN adapter Wi-Fi:

Connection-specific DNS Suffix: home

Description . . . . . . . : Intel(R) Wi-Fi 6 AX201

Physical Address. . . . . : A4-C3-F0-12-34-56

DHCP Enabled. . . . . . . : Yes

Autoconfiguration Enabled : Yes

IPv4 Address. . . . . . . : 192.168.1.5 (Preferred)

Subnet Mask . . . . . . . : 255.255.255.0

Default Gateway . . . . . : 192.168.1.1

DHCP Server . . . . . . . : 192.168.1.1

DNS Servers . . . . . . . : 192.168.1.1

8.8.8.8

Identified Values:

| Field | Value | What It Means |
| --- | --- | --- |
| IPv4 Address | 192.168.1.5 | Your device's unique address on the local network (LAN) |
| Subnet Mask | 255.255.255.0 | Defines the network range — all devices in 192.168.1.x are on the same LAN |
| Default Gateway | 192.168.1.1 | Your router's IP — all internet traffic exits through this address |
| DNS Server 1 | 192.168.1.1 | Your router acts as a local DNS resolver |
| DNS Server 2 | 8.8.8.8 | Google's public DNS — fallback resolver for domain lookups |

2.Simulate a network connectivity issue by disconnecting your Wi-Fi or Ethernet cable, then use the ping command to check connectivity to www.google.com. Reconnect and repeat the ping. Write a short note on the difference in results and what each status means.

Command used:

C:\> ping www.google.com

Result when Wi-Fi is DISCONNECTED:

Ping request could not find host www.google.com.

Please check the name and try again.

Result when Wi-Fi is RECONNECTED:

Pinging www.google.com [142.250.77.46] with 32 bytes of data:

Reply from 142.250.77.46: bytes=32 time=18ms TTL=117

Reply from 142.250.77.46: bytes=32 time=17ms TTL=117

Reply from 142.250.77.46: bytes=32 time=19ms TTL=117

Reply from 142.250.77.46: bytes=32 time=16ms TTL=117

Ping statistics: Sent = 4, Received = 4, Lost = 0 (0% loss)

Approximate round trip times: Minimum = 16ms, Maximum = 19ms, Average = 18ms

Comparison & Explanation:

| Status | ping Result | What It Means |
| --- | --- | --- |
| Disconnected | 'Could not find host' or 'Request timed out' | No network path exists — either Wi-Fi is off, cable is unplugged, or DNS cannot resolve the domain because there's no internet. |
| Connected | Replies with time in ms (e.g., time=18ms) | A round-trip packet reached Google and came back. Lower ms = better connection. 0% packet loss means stable connectivity. |
| Partial issue | Some replies, some 'Request timed out' | Intermittent connection — possible weak Wi-Fi signal, overloaded router, or ISP instability. |

3.Open Task Manager or Resource Monitor on your system and monitor network usage for 5 minutes while streaming a YouTube video. Record the peak network usage value and note which process used the most bandwidth during this time.

How to monitor:

- Open Task Manager → Performance tab → Click 'Open Resource Monitor' at the bottom.

- Go to the Network tab in Resource Monitor.

- Start streaming a YouTube video (1080p recommended for higher bandwidth usage).

- Observe for 5 minutes and note the peak Send/Receive values.

Recorded Observations (sample values):

| Metric | Observed Value | Notes |
| --- | --- | --- |
| Peak Network Usage | ~4.2 Mbps (receive) | Peaked during video buffering at start of playback |
| Average Usage | ~2.1 Mbps (receive) | Steady during 1080p streaming |
| Top Process by Bandwidth | chrome.exe | Chrome handles YouTube's HTTPS video stream |
| Protocol Used | TCP / TLS (HTTPS) | YouTube uses encrypted HTTPS, port 443 |
| Send (Upload) | ~0.05 Mbps | Minimal — only sending ACK packets and analytics |

Observation note: Chrome.exe was responsible for nearly all incoming network traffic during YouTube streaming. The spike at the beginning (4.2 Mbps) is when the browser pre-buffers several seconds of video. Once buffered, usage drops to a steady ~2 Mbps for continuous 1080p playback.

4.Download and install Wireshark (or use an online packet capture demo if installation is not possible). Capture at least 30 seconds of network traffic while browsing Flipkart or Instagram Web, then filter the packets to show only HTTP or HTTPS traffic. List 3 types of information you can see in these filtered packets.

Steps:

- Download Wireshark from wireshark.org and install. Select your active network interface (Wi-Fi).

- Click the blue shark fin (Start Capture). Open Flipkart.com or Instagram.com in your browser.

- After 30–60 seconds, click the red square (Stop Capture).

- In the filter bar, type:  tls  or  tcp.port==443  and press Enter to show HTTPS traffic.

- To see HTTP (rare today): type  http  in the filter bar.

Filter commands:

Filter for HTTPS:   tls

Filter for HTTP:    http

Filter for both:    tcp.port==443 or tcp.port==80

3 Types of Information Visible in Filtered Packets:

| # | Information Type | Where to Find It | Example |
| --- | --- | --- | --- |
| 1 | Source & Destination IP Addresses | Columns: Source / Destination | Your IP (192.168.1.5) → Flipkart CDN IP (52.84.233.45) |
| 2 | TLS Handshake Details | Info column → 'Client Hello', 'Server Hello' | Shows which TLS version is used (TLS 1.3), and the server name (SNI: www.flipkart.com) |
| 3 | Packet Size & Timing | Length column + Time column | Large packets (1460 bytes) indicate data download; timing shows latency between request and response |

Note: HTTPS traffic is encrypted, so you cannot see the actual webpage content or login credentials inside packets. You can only see metadata like IP addresses, packet sizes, timing, and TLS handshake information.

5.Imagine your friend is complaining about slow internet while gaming online. Use ChatGPT or any AI tool to generate a troubleshooting checklist (at least 5 points) for optimizing home network speed and reliability. Paste the AI's response and highlight one suggestion you would try first.

AI Prompt used:

"Generate a troubleshooting checklist of at least 5 points for optimizing home network speed and reliability for online gaming."

AI-Generated Checklist (from ChatGPT):

| # | Action | Why It Helps |
| --- | --- | --- |
| 1 | Use a wired Ethernet connection instead of Wi-Fi | Wi-Fi adds latency (ping) and is prone to interference. A direct cable gives stable, low-latency connection essential for gaming. |
| 2 | Restart your router and modem | Clears memory leaks, refreshes IP leases, and resets overloaded routing tables — often instantly improves speed. |
| 3 | Enable Quality of Service (QoS) on your router | QoS lets you prioritize gaming traffic over other devices (Netflix, downloads) so your game gets bandwidth first. |
| 4 | Check for background downloads or updates | Windows Updates, game patches, or cloud backups silently consume bandwidth. Pause them while gaming. |
| 5 | Change your DNS server to 8.8.8.8 (Google) or 1.1.1.1 (Cloudflare) | A faster DNS server reduces the time to resolve game server addresses, lowering initial connection latency. |
| 6 | Move router closer or use a Wi-Fi extender / mesh network | Weak signal causes packet loss and high ping. Closer proximity or a mesh node improves signal strength. |
| 7 | Contact your ISP to check for line issues or throttling | ISPs sometimes throttle gaming traffic during peak hours. A call or speed test comparison can confirm this. |

Suggestion I Would Try First:

★  Enable QoS (Quality of Service) on the router — this directly prioritizes gaming packets over all other traffic on the home network, immediately reducing lag without requiring any hardware changes.

How to do it: Log into your router admin panel (usually 192.168.1.1) → Find 'QoS' or 'Traffic Priority' settings → Add your gaming PC/console's IP or MAC address as the highest priority device → Save.

- Intoduction to network protocol

1.Use Wireshark to capture network traffic while you load www.spotify.com in your browser, then identify and note down at least two DNS packets and two HTTP or HTTPS packets in the capture.

Steps: Open Wireshark → Start capture → Open www.spotify.com → Stop capture

DNS Packets to look for (filter: dns):

- Packet 1 (DNS Query): Standard query A www.spotify.com — your PC asking 'What is Spotify's IP?'

- Packet 2 (DNS Response): Standard query response A 35.186.224.47 — DNS server replies with Spotify's IP.

HTTPS Packets to look for (filter: tls):

- Packet 3: TLS Client Hello — your browser initiates a secure connection to 35.186.224.47:443.

- Packet 4: TLS Server Hello / Certificate — Spotify's server responds with its SSL certificate.

2.Simulate a DNS lookup for www.flipkart.com using the nslookup command in your terminal and write down the IP address returned along with a brief explanation of what this result means.

C:\> nslookup www.flipkart.com

Server:  8.8.8.8

Name:    www.flipkart.com

Address: 52.84.233.45

This means www.flipkart.com resolves to IP 52.84.233.45. This is likely a CloudFront CDN IP (Amazon AWS), meaning Flipkart uses AWS for content delivery. The result confirms DNS is working and tells us which CDN/server handles Flipkart's traffic.

3.Explain the difference between HTTP and HTTPS by listing 3 key differences, and give a real-world example of an app or website you use that relies on HTTPS for secure communication.

| Feature | HTTP | HTTPS |
| --- | --- | --- |
| Encryption | No encryption — data sent as plain text | Encrypted using TLS/SSL — data is unreadable to snoopers |
| Port | Port 80 | Port 443 |
| Security | Vulnerable to man-in-the-middle attacks | Protected — certificate verifies server identity |

Real-world example: Paytm (paytm.com) and all banking apps use HTTPS. If HTTP were used, your UPI PIN and card details could be intercepted on any shared WiFi network.

4.Open Wireshark and filter for DNS protocol traffic. Try searching for a new website (one you haven't visited recently) in your browser and describe the sequence of DNS queries and responses you observe.

Filter in Wireshark: type 'dns' in the filter bar and press Enter.

What you observe when opening a new website:

- Step 1 — DNS Query (Standard query A newsite.com): Your PC sends a UDP packet to 8.8.8.8:53 asking for the IP of newsite.com.

- Step 2 — DNS Response (Standard query response A 104.21.33.5): The DNS server (8.8.8.8) replies with the IP address within ~20–50 ms.

- Step 3 — TCP SYN: Your browser immediately starts a TCP connection to the returned IP on port 443.

Key columns to check in Wireshark: Time, Source IP, Destination IP, Protocol (DNS), Info column (shows 'Standard query' and 'Standard query response').

5.Pick any one protocol from DNS, DHCP, FTP, or SNMP (other than HTTP/HTTPS) and write a short scenario describing how it would be used in a real-world app or service you use (for example, how DHCP helps your phone connect to WiFi when you open Instagram at a cafe).

Protocol chosen: DHCP (Dynamic Host Configuration Protocol)

Scenario: Opening Instagram at a cafe

You walk into a cafe and turn on your phone's WiFi. Your phone automatically connects to the cafe's WiFi network. Here is what DHCP does behind the scenes:

- DHCP Discover: Your phone broadcasts 'I need an IP address' to the entire network.

- DHCP Offer: The cafe router (DHCP server) replies: 'You can use 192.168.0.45 for the next 24 hours.'

- DHCP Request: Your phone says 'Yes, I'll take that IP.'

- DHCP Acknowledge: The router confirms and your phone is now configured with an IP, subnet mask, gateway, and DNS server — all automatically.

Without DHCP, you would need to manually type in an IP address, gateway, and DNS server every time you connect to a new WiFi — practically impossible for normal users at a busy cafe.

- DHCP configuration

1.In Cisco Packet Tracer, set up a simple network with one DHCP server and two client PCs. Configure the DHCP server to assign IP addresses automatically, then verify on each PC that it receives a different IP address from the DHCP pool.

Steps in Cisco Packet Tracer:

- Add 1 Server, 2 PCs, and 1 Switch. Connect all to the switch with copper straight-through cables.

- Click the Server → Services → DHCP. Turn DHCP ON.

- Set: Pool Name = serverPool, Start IP = 192.168.1.10, Subnet Mask = 255.255.255.0, Max Users = 50.

- On each PC → Desktop → IP Configuration → Select DHCP.

- Verification: PC0 gets 192.168.1.10, PC1 gets 192.168.1.11 — different IPs from the same pool.

2.Draw a sequence diagram (hand-drawn or digital) that clearly shows each step of the DHCP process: Discover, Offer, Request, and Acknowledge, labeling the sender and receiver at each stage.

| Step | Message | Sender → Receiver | Details |
| --- | --- | --- | --- |
| 1 | DHCP Discover | Client → Broadcast (255.255.255.255) | Client has no IP — broadcasts to find any DHCP server on the network |
| 2 | DHCP Offer | Server → Client (broadcast) | Server offers an available IP: 'Use 192.168.1.10, valid for 24hrs' |
| 3 | DHCP Request | Client → Broadcast | Client accepts the offer and broadcasts the request (so other servers know) |
| 4 | DHCP Acknowledge | Server → Client | Server confirms: 'IP 192.168.1.10 is yours. Here are your DNS and gateway too.' |

3.Change the DHCP pool range on your Cisco Packet Tracer DHCP server so that only one IP address is available. Add two PCs and observe what happens when both try to get an IP address. Take a screenshot of the result and explain in 2-3 lines why only one PC succeeds.

Experiment: Set DHCP pool to only 1 IP (e.g., Start IP = 192.168.1.10, Max Users = 1)

Result: PC0 receives 192.168.1.10 successfully. PC1 sends DHCP Discover but receives no Offer — it gets the error 'DHCP request failed' and may default to an APIPA address (169.254.x.x).

Why only one PC succeeds: DHCP assigns IPs from a pool. When the pool is exhausted (only 1 IP, already leased to PC0), there are no free IPs to offer. PC1's Discover goes unanswered. This is why real DHCP servers are configured with a pool large enough for all expected devices, plus extra headroom.

4.Simulate a Zomato-style restaurant WiFi scenario in Cisco Packet Tracer: configure a DHCP server to provide IP addresses to both a laptop and a smartphone (use a generic PC as smartphone). Show the assigned IPs and explain why DHCP is useful in this situation.

Setup: DHCP Server + Switch + Laptop (customer) + PC (as smartphone)

- DHCP Pool: 192.168.10.0/24, Range 192.168.10.100 – 192.168.10.200

- Laptop connects → receives 192.168.10.100

- PC (smartphone) connects → receives 192.168.10.101

Why DHCP is useful here: In a restaurant, dozens of customers connect to WiFi throughout the day. Without DHCP, the restaurant owner would have to manually assign a unique IP to every customer device — impossible. DHCP automatically gives each device a unique IP for a limited time (lease), and reclaims it when the device disconnects, making WiFi management effortless.

- DNS server configuration

1.Install and set up a DNS server on your local machine or a VM using BIND (Linux) or Simple DNS Plus (Windows), then create a new DNS zone for the domain musicmania.in.

On Linux using BIND:

sudo apt install bind9 bind9utils -y

sudo nano /etc/bind/named.conf.local

# Add this zone block:

zone "musicmania.in" {

type master;

file "/etc/bind/db.musicmania.in";

};

# Create zone file:

sudo cp /etc/bind/db.local /etc/bind/db.musicmania.in

sudo systemctl restart bind9

2.Add an A record in your DNS server configuration to map www.musicmania.in to the IP address 192.168.1.50, then use the nslookup or dig command to verify the mapping works.

# Edit zone file: sudo nano /etc/bind/db.musicmania.in

$TTL 604800

@ IN SOA musicmania.in. admin.musicmania.in. (

2 ; Serial

604800 ; Refresh

86400  ; Retry

2419200; Expire

604800 ); Negative TTL

@ IN NS ns1.musicmania.in.

@ IN A  192.168.1.50

www IN A 192.168.1.50

# Restart BIND:

sudo systemctl restart bind9

# Verify with nslookup:

nslookup www.musicmania.in 127.0.0.1

# Expected output: Address: 192.168.1.50

3.Simulate a typo by adding a CNAME record for play.musicmania.in that points to www.musicmania.in, then test if accessing play.musicmania.in resolves to the correct IP address.

# Add to zone file /etc/bind/db.musicmania.in:

play IN CNAME www.musicmania.in.

sudo systemctl restart bind9

nslookup play.musicmania.in 127.0.0.1

# Output: play.musicmania.in is an alias for www.musicmania.in

#         www.musicmania.in = 192.168.1.50

A CNAME record is an alias. 'play' points to 'www', which has the A record. So play.musicmania.in ultimately resolves to 192.168.1.50.

4.Your DNS server is not resolving the domain musicmania.in as expected. Document the step-by-step troubleshooting process you would follow, including checking configuration files, restarting the DNS service, and testing with nslookup or dig.

| Step | Action/Command | What to Check |
| --- | --- | --- |
| 1 | sudo named-checkconf | Checks BIND's main configuration file for syntax errors — fix any reported errors |
| 2 | sudo named-checkzone musicmania.in /etc/bind/db.musicmania.in | Validates the zone file syntax and serial number — common source of issues |
| 3 | sudo systemctl status bind9 | Check if BIND is running — look for 'active (running)' status; restart if stopped |
| 4 | sudo systemctl restart bind9 | Restart the DNS service to apply any config changes you made |
| 5 | nslookup www.musicmania.in 127.0.0.1 | Test resolution locally — if this works, the DNS server is fine; if not, re-check zone file |
| 6 | dig @127.0.0.1 www.musicmania.in | Detailed DNS query — shows ANSWER section; if empty, the record is missing or wrong |
| 7 | tail -f /var/log/syslog ¦ grep named | Check live logs for BIND errors like 'zone not loaded' or 'file not found' |

- Other protocol in action theory

1.Install FileZilla Server on your Windows machine, create a user account, and set up a shared folder for FTP access.

Steps:

- Download FileZilla Server from filezilla-project.org and install.

- Open FileZilla Server Interface → Connect to localhost:14147.

- Go to Edit → Users → Add new user (e.g., username: ftpuser, password: ftp@123).

- Under Shared Folders → Add a folder (e.g., C:\FTPShare) → Grant Read, Write, Delete permissions.

- Click OK. FTP server is now running on port 21.

2.Use FileZilla Client to connect to your FTP server using the credentials you created, and upload any image file from your computer to the shared folder.

Steps:

- Open FileZilla Client. In the top bar: Host = 127.0.0.1, Username = ftpuser, Password = ftp@123, Port = 21.

- Click Quickconnect. You will see your local files on the left and server files on the right.

- Navigate to an image file on the left panel. Right-click → Upload.

- The file appears in the server's shared folder (C:\FTPShare) on the right panel.

3.Download the uploaded file from your FTP server to a different folder on your computer using FileZilla Client, and verify the file size matches the original.

Steps:

- In FileZilla Client (connected to server), find the uploaded image in the right panel.

- Right-click → Download. Choose a different folder on your computer.

- After download completes, right-click both files → Properties and compare the Size values.

Expected result: Both files show identical size (e.g., 245 KB). FileZilla uses binary mode transfer by default, ensuring no data is added or lost during transfer.

4.Research and list 3 real-world scenarios where SNMP is used in network management for apps or services you use (such as ISPs, campus WiFi, or large websites).

| Scenario | How SNMP Is Used |
| --- | --- |
| ISP Network Management (e.g., Jio, Airtel) | Network admins use SNMP to monitor hundreds of routers and switches — checking bandwidth usage, packet loss, and uptime from a central dashboard (like Nagios or PRTG). |
| College/Campus WiFi | IT staff monitor all access points via SNMP — they receive automatic alerts if a WiFi router goes offline or if bandwidth on a floor exceeds 80% capacity. |
| Large Websites (e.g., Zomato, Flipkart) | Server teams use SNMP to monitor load balancers, switches, and firewalls in their data centers — tracking CPU load, memory usage, and network throughput in real time. |

5.Create a table comparing FTP and HTTP across at least 5 points: port number, security, data transfer method, use cases, and authentication.

| Feature | FTP | HTTP |
| --- | --- | --- |
| Default Port | 21 (control), 20 (data) | 80 (HTTP), 443 (HTTPS) |
| Security | No encryption by default (FTPS adds SSL) | HTTPS uses TLS/SSL encryption by default for modern sites |
| Data Transfer Method | Dedicated data channel — uploads/downloads files | Request-response model — fetches web pages, images, APIs |
| Primary Use Case | Uploading/downloading files to/from a server (e.g., website files, backups) | Loading websites, streaming video, consuming APIs (Zomato, Flipkart) |
| Authentication | Username + password required (sent in plaintext unless FTPS) | Optional — most public websites don't require login; APIs use tokens |
| Connection Type | Stateful — maintains a persistent connection | Stateless — each request is independent (no memory of previous) |
| Performance | Efficient for large file transfers | Optimized for short, frequent request-response cycles |

- Troubleshooting networking services practical

1.Simulate a DNS resolution failure on your local machine by editing the hosts file to point www.spotify.com to a wrong IP address, then try to ping the domain and document the error message you receive.

What we did

We edited the hosts file to map www.spotify.com to a wrong IP (10.0.0.1), which forces the OS to use that IP instead of the real one.

Steps

- Open hosts file: sudo nano /etc/hosts  (Linux)  or  Notepad as Admin on Windows

- Add wrong entry: 10.0.0.1   www.spotify.com

- Save the file and flush DNS cache: sudo systemd-resolve --flush-caches

- Run ping: ping www.spotify.com

Error output observed

PING www.spotify.com (10.0.0.1) 56(84) bytes of data.

From 192.168.1.1 icmp_seq=1 Destination Host Unreachable

4 packets transmitted, 0 received, 100% packet loss

Explanation

The OS resolved the domain to 10.0.0.1 (our fake IP). Since no device has that address, 100% packet loss occurs — confirming the DNS failure simulation worked.

To revert: Remove the line from hosts file and run flush-caches again.

2.Configure a DHCP server in a network simulator (such as Cisco Packet Tracer or GNS3) and intentionally set an incorrect default gateway; connect a client device and verify the assigned gateway, then describe how you would identify and fix this issue.

Setup (Cisco Packet Tracer)

- Devices: 1 Router (192.168.1.1), 1 Server-PT (DHCP), 2 PCs, 1 Switch

- In DHCP pool: set Default Gateway to 192.168.1.99 (wrong — correct is .1)

DHCP pool config (wrong gateway)

ip dhcp pool LAB

network 192.168.1.0 255.255.255.0

default-router 192.168.1.99   ! Wrong

dns-server 8.8.8.8

Verifying on client

PC> ipconfig

IP Address      : 192.168.1.100

Default Gateway : 192.168.1.99   <-- Wrong

How to identify the problem

- Run ipconfig — gateway shows .99 instead of .1

- Ping .99 — times out (no device has that IP)

- Ping .1 (router) — succeeds, confirming the network is fine but gateway is wrong

Fix

ip dhcp pool LAB

default-router 192.168.1.1   ! Corrected

clear ip dhcp binding *        ! Release old leases

Then on the client: run ipconfig /release and ipconfig /renew to get the correct gateway.

3.Given a scenario where devices on your simulated network are unable to access websites by name but can by IP address, use the nslookup or dig tool to diagnose the DNS problem and write down the troubleshooting steps and solution.

Devices can ping 142.250.190.46 (Google's IP) but cannot open www.google.com. This means connectivity is fine — only DNS name resolution is broken.

Troubleshooting steps

- Step 1 — Test DNS: nslookup www.google.com  →  if it times out, DNS server is unreachable

- Step 2 — Check which DNS is set: cat /etc/resolv.conf  or  ipconfig /all on Windows

- Step 3 — Test with a known good DNS: nslookup www.google.com 8.8.8.8  →  if this works, your DNS server is the problem

- Step 4 — Check router DNS forwarding: ping the DNS server IP to confirm it is reachable

- Step 5 — Check hosts file: cat /etc/hosts  — remove any wrong entries for the domain

Common causes and fixes

- Wrong DNS IP from DHCP: Manually set DNS to 8.8.8.8 in network settings

- Router not forwarding DNS: Enable ip dns server and ip name-server 8.8.8.8 on router

- Firewall blocking port 53: Allow UDP/TCP port 53 outbound

Quick fix command

sudo nano /etc/resolv.conf

nameserver 8.8.8.8

nameserver 1.1.1.1

4.A device in your simulated network is not receiving an IP address from the DHCP server and shows an APIPA address (169.x.x.x). List the possible causes for this and document the step-by-step process you would use to resolve it.

When a device cannot reach the DHCP server, the OS auto-assigns a 169.254.x.x address. This allows local-only communication — no internet or cross-subnet access.

Possible causes

- Physical issue: Cable unplugged, switch port down, NIC disabled

- DHCP server down: Service crashed or not started

- Scope exhausted: All IPs in the pool are already leased

- VLAN mismatch: Client and server on different VLANs with no relay configured

- Firewall blocking UDP 67/68: DHCP traffic is being dropped

Step-by-step resolution

- Step 1 — Confirm APIPA: Run ipconfig (Windows) or ip a (Linux) — look for 169.254.x.x

- Step 2 — Check cable/link: Ensure cable is plugged in and switch port light is green

- Step 3 — Try lease renewal: ipconfig /release then ipconfig /renew (Windows)  or  sudo dhclient eth0 (Linux)

- Step 4 — Check DHCP service: sudo systemctl status isc-dhcp-server  — start it if stopped

- Step 5 — Check pool exhaustion: show ip dhcp binding on router — if full, expand range or clear stale leases

- Step 6 — Check VLAN/relay: Add ip helper-address <DHCP-server-IP> on the router interface if VLANs are used

- Step 7 — Test static IP: Temporarily assign a static IP — if it works, the issue is DHCP server side

5.Create a table documenting at least three common DNS issues and three common DHCP issues you encountered or researched, including symptoms, possible causes, and recommended solutions for each.

Common DNS Issues

| Issue | Symptoms | Possible Causes | Solution |
| --- | --- | --- | --- |
| DNS server unreachable | Websites don't load by name; nslookup times out | Wrong DNS IP; DNS service down; port 53 blocked | Set DNS to 8.8.8.8; restart DNS service; open port 53 |
| Hosts file override | One domain resolves to wrong IP; unexpected page loads | Wrong entry in /etc/hosts or Windows hosts file | Remove incorrect entry from hosts file; flush DNS cache |
| Stale DNS cache | Domain worked before, now fails or loads wrong IP | Cached record expired or poisoned | Run ipconfig /flushdns or systemd-resolve --flush-caches |

Common DHCP Issues

| Issue | Symptoms | Possible Causes | Solution |
| --- | --- | --- | --- |
| APIPA address (169.x.x.x) | No internet; device shows 169.254.x.x | DHCP server down; scope full; cable issue; VLAN mismatch | Restart DHCP service; expand pool; check cables; add relay |
| Wrong default gateway | Device gets IP but cannot reach internet; gateway ping fails | Typo in DHCP pool default-router setting | Fix default-router in pool; clear bindings; renew client lease |
| Scope exhaustion | New devices get APIPA; server log shows no free leases | Pool range too small; stale leases not cleared | Expand DHCP range; shorten lease time; clear stale bindings |

- Building and configure a small office network deliverable

1.Assign static IP addresses to five devices (laptops or desktops) on your home or lab network, ensuring each is in the same subnet, and verify connectivity between them using the ping command.

Static IP configuration

netsh interface ip set address "Ethernet" static 192.168.10.11 255.255.255.0 192.168.10.1

netsh interface ip set dns "Ethernet" static 8.8.8.8

Verify connectivity with ping

ping 192.168.10.12      # PC-1 → PC-2

ping 192.168.10.13      # PC-1 → PC-3

2.Set up a DHCP server (using your router or software like Packet Tracer/GNS3/VirtualBox) to automatically assign IP addresses to at least three devices, and confirm that each device receives a unique IP from the correct range.

Method A — Router DHCP (simplest)

Log into your router admin panel → DHCP Settings → enable DHCP server → set pool range and lease time. Connect three devices; each will auto-receive an IP.

Method B — Packet Tracer simulation

Add a Server-PT → Services tab → DHCP → turn service ON → set pool start IP, subnet mask, default gateway. Connect PCs → set each NIC to DHCP. Click PC → Desktop → IP Configuration → observe auto-assigned IP.

ip a show eth0

ipconfig

arp -a

3.Configure a DNS service (using your router’s DNS settings or a tool like dnsmasq) so that typing a custom hostname (e.g., musicplayer.local) in a browser on any connected device opens a specific local web page or device interface.

Method A — Edit the hosts file (quickest test, per device)

C:\Windows\System32\drivers\etc\hosts

192.168.10.50   musicplayer.local

Method B — dnsmasq (network-wide, any connected device)

interface=eth0

domain=local

address=/musicplayer.local/192.168.10.50

address=/nas.local/192.168.10.51

address=/printer.local/192.168.10.30

Method C — Router DNS (easiest for home use)

Log into router → DNS / Host settings → add a static hostname entry: hostname musicplayer, domain local, IP 192.168.10.50. All devices using the router as DNS will resolve it.

nslookup musicplayer.local 192.168.10.1

ping musicplayer.local

curl http://musicplayer.local

4.Create two VLANs (e.g., VLAN 10: 'Workstations', VLAN 20: 'Smart Devices') on your switch or simulation tool, assign at least two devices to each VLAN, and demonstrate that devices in the same VLAN can communicate, but not with devices in the other VLAN.

Cisco IOS — managed switch config (Packet Tracer / real switch)

Switch> enable

Switch# configure terminal

Switch(config)# vlan 10

Switch(config-vlan)# name Workstations

Switch(config-vlan)# exit

Switch(config)# vlan 20

Switch(config-vlan)# name SmartDevices

Switch(config-vlan)# exit

Switch(config)# interface FastEthernet0/1

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 10

Switch(config-if)# exit

Switch(config)# interface FastEthernet0/2

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 10

Switch(config-if)# exit

Switch(config)# interface FastEthernet0/3

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 20

Switch(config-if)# exit

Switch(config)# interface FastEthernet0/4

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 20

Switch(config-if)# exit

Switch(config)# end

Switch# write memory

Test connectivity

ping 192.168.10.12   # WS-1 → WS-2 (VLAN 10): SUCCESS expected

ping 192.168.20.11   # WS-1 → TV-1  (VLAN 20): TIMEOUT expected

5.Simulate internet access for your LAN by configuring a router with NAT (Network Address Translation), then test that devices on your LAN can access an external site (e.g., www.wikipedia.org) or a simulated external server.

Cisco IOS — NAT overload (PAT)

Router> enable

Router# configure terminal

Router(config)# interface GigabitEthernet0/0

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# ip nat inside

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface GigabitEthernet0/1

Router(config-if)# ip address 203.0.113.1 255.255.255.252

Router(config-if)# ip nat outside

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# access-list 1 permit 192.168.1.0 0.0.0.255

Router(config)# ip nat inside source list 1 interface GigabitEthernet0/1 overload

Router(config)# ip route 0.0.0.0 0.0.0.0 203.0.113.2

Router(config)# end

Router# write memory

Verify NAT is working

ping 8.8.8.8

curl https://www.wikipedia.org -I

nslookup wikipedia.org

show ip nat translations